“Power Consumption Analysis and Load Prediction in Electric Vehicle Traffic: A SUMO-Based Simulation on Custom Road Networks”

THESIS REPORT

Chapter 1: Introduction

1.1 Background

The rapid rise of electric vehicles is reshaping transportation and power distribution systems. Unlike traditional combustion vehicles, EVs shift energy demand from fuel stations to the electrical grid, creating spatially and temporally varying loads. Understanding EV movement, energy usage, and charging behavior is essential for load forecasting, grid reliability, and optimizing charging infrastructure.

Traffic simulation tools like SUMO help represent real urban mobility, while EV extensions in SUMO allow modeling battery behavior, charging events, and energy consumption. Integrating these outputs with load prediction models forms a powerful approach for analyzing energy impacts of EV adoption.

1.1.1 Definition

- Electric Vehicle (EV): A vehicle powered by rechargeable batteries and electric motors.

- Load Prediction: Estimating future electrical demand based on consumption patterns.

- Traffic Microsimulation: Vehicle-level movement simulation capturing speed, acceleration, and routing behavior, power consumption, load analysis.

- State of Charge (SOC): Battery charge level representing remaining energy.

1.2 Problem Statement

Bangladesh is gradually preparing for EV adoption, but lacks detailed modeling tools for understanding how EV traffic will affect future electrical loads. Without proper forecasting, charging stations may be overloaded or underutilized. There is no simulation-based framework that integrates:

- Realistic movement of EVs

- Dynamic battery consumption

- Charging behavior at nodes

- Load estimation for different routes

Hence, a research model is needed to simulate EV mobility and predict power demand.

1.3 Objectives

- To construct a custom transportation network using SUMO/NETEDIT.

- To simulate EV movement using defined routes and traffic demand.

- To integrate charging stations at every node and track EV battery behavior.

- To compute a travel-cost matrix (distance, energy usage, load).

- To develop a model for EV load prediction based on simulation outputs.

- To identify high-load nodes and propose optimization strategies.

1.4 Research Questions

- How does EV mobility across a road network translate into electrical power demand?

- Which nodes experience the highest charging load?

- Can simulation-based features predict future EV charging requirements?

- How do route selection and traffic density influence energy consumption?

1.5 Significance of the Study

This research benefits:

• Power grid planners and distribution utilities

For forecasting real-time and future EV charging demand, enabling:

- Improved load balancing

- Prevention of local transformer overloading

- Strategic placement of charging stations

- Better scheduling of peak vs off-peak charging periods

• Urban traffic engineers

To optimize EV routing strategies considering:

- Road congestion levels

- Energy-efficient driving paths

- Travel cost minimization using real-time traffic simulation

• Charging station operators & infrastructure investors

Supporting:

- Determination of optimal station locations

- Estimating future power draw requirements

- Predicting revenue & operational load

- Scaling charging capacity based on local EV flow patterns

• Environmental sustainability & climate initiatives

By promoting:

- Efficient energy utilization

- Minimization of charger-idling losses

Chapter 2: Contextual Review and Research Gaps

2.1 Literature Review

The rapid growth of electric vehicles (EVs) has led to increasing global attention on power demand, charging behavior, and their implications for urban energy systems, especially in developing countries. Existing research on EV charging patterns broadly addresses four areas: power consumption characteristics, charging behavior and traffic-flow simulations, SUMO-based studies for route–energy coupling, and load forecasting for smart grid integration. Studies such as “Prediction of EV Charging Behavior using Machine Learning” (IEEE, 2017) explored how EV charging durations and energy consumption fluctuate based on user behavior, time-of-day, and contextual factors, highlighting that uncontrolled EV charging significantly stresses local power grids. Similarly, “Demand Forecasting for Electric Vehicle Charging Stations using Multivariate Time-Series Analysis” (arXiv, 2025) emphasized that charging demand exhibits strong temporal patterns influenced by environmental and socioeconomic conditions—factors highly relevant for densely populated Bangladeshi cities where energy shortages and peak-hour pressure are frequent. Research focusing on EV load characteristics, such as “Electric Vehicle Charging Load Prediction Using a Novel Machine Learning-Based Technique” (SDEWES Conference, 2023) and “Electric Vehicle Charging Load Forecasting: A Comparative Study of Deep Learning Approaches” (Energies, 2019), consistently concluded that EV charging is inherently stochastic, varies with user groups, and can cause operational instability in urban grids without proper planning. On the simulation side, papers like “Dynamic Load Balancing for EV Charging Stations Using Reinforcement Learning” (arXiv, 2025) and “Data-Driven Modeling of Electric Vehicle Charging Sessions” (World Electric Vehicle Journal, 2025) demonstrated the usefulness of integrating mobility patterns with energy consumption using tools such as SUMO, traffic demand models, and energy-aware routing policies. These works highlight that accurate modeling of movement paths, charging intervals, and grid interactions is essential for designing reliable EV charging networks. SUMO-based research also underlined the importance of travel path estimation and node–edge energy mapping for understanding real-world charging impacts—critical insights for emerging EV markets like Bangladesh where structured datasets and predictive infrastructure planning are limited. Smart-grid-oriented studies, including “Electric Vehicle Charging Load Prediction Considering Spatio-Temporal Node Importance Information” (Energies, 2024) and “Machine Learning Optimization for Hybrid Electric Vehicle Charging” (Scientific Reports, 2024), stressed that EV integration must consider grid limitations, geographic constraints, and the distribution of charging stations. These papers highlighted that countries with weaker grid resilience—similar to Bangladesh—require predictive and simulation-backed planning before large-scale EV adoption to prevent transformer overload, voltage drops, and unplanned outages. Collectively, the reviewed literature establishes that EVs significantly influence local energy systems and that accurate forecasting, simulation, and infrastructure design are crucial for sustainable EV deployment. Despite numerous global studies, Bangladesh-specific research remains extremely limited, especially in the context of battery-powered vehicles (BPVs) such as easy bikes, e-rickshaws, and e-vans that dominate the informal transport sector. These BPVs place a heavy load on local distribution transformers and frequently charge during peak hours, posing challenges for already stressed urban grids.

2.2 Gap Analysis & Research Novelty

Although extensive international research has explored electric vehicle (EV) charging demand, power consumption behavior, and grid impacts, significant gaps remain in applying these findings to developing countries—particularly Bangladesh—where battery-powered vehicles (BPVs) such as easy bikes, e-rickshaws, and small-scale EVs dominate the transportation sector and heavily influence local electricity networks. Existing studies, such as IEEE 2017: Prediction of EV Charging Behavior using Machine Learning, arXiv 2025: Demand Forecasting for Electric Vehicle Charging Stations using Multivariate Time-Series Analysis, and 2023 Electric Vehicle Charging Demand Prediction Using a Novel Machine Learning-Based Technique, primarily examine structured EV charging ecosystems with stable grids, formal charging stations, and standardized vehicle types—conditions that do not represent Bangladesh’s informal, unregulated BPV charging practices. Many advanced works, including 2024 Electric Vehicle Charging Load Prediction Considering Spatio-Temporal Node Importance, World Electric Vehicle Journal 2025: Data-Driven Modeling of Electric Vehicle Charging Sessions, and Energies 2019: EV Load Forecasting Using Hybrid CNN-LSTM-AM, highlight the importance of spatio-temporal charging patterns, urban load variations, and infrastructure-aware planning; however, these studies rely on rich datasets from developed infrastructures (USA, China, EU) and overlook unique load stresses created by BPVs that charge directly from neighborhood transformers in Bangladesh. Similarly, works like Dynamic Load Balancing for EV Charging Stations Using Reinforcement Learning and Scientific Reports 2024: Machine Learning Optimization for Hybrid EV Charging address grid coordination, pricing control, and smart-grid supported charging systems—features that Bangladesh currently lacks due to the absence of nation-wide EV charging standards. Despite the breadth of global research, almost no study evaluates real-time BPV movement-based power consumption, charging load impacts on congested urban feeders, or micro-level transformer stress arising from unplanned charging clusters—issues highly relevant to cities like Dhaka and Jashore. Given this gap, the novelty of this research lies in developing a SUMO-based EV mobility and power consumption simulation framework customized for Bangladesh, integrating custom road networks, traffic flows, and BPV charging behaviors to estimate charging load peaks, transformer stress patterns, and urban energy demand fluctuations. Unlike previous works that rely on sophisticated, well-instrumented charging infrastructures, this study uniquely focuses on Bangladesh’s informal BPV ecosystem, low-voltage distribution grid characteristics, and urban traffic-induced charging variability, providing one of the first comprehensive attempts to link EV mobility dynamics with realistic power consumption forecasting for developing-country contexts. This makes the research not only original but also essential for future planning of charging station placement, urban load management, and national BPV energy policy formation in Bangladesh.

Chapter 3: Methodology

3.1 Research Design

This study adopts a simulation-driven predictive modeling framework to forecast electric vehicle (EV) charging demand and load distribution across a transportation network. The methodology integrates traffic simulation, EV energy modeling, and machine learning-based forecasting. The framework consists of the following stages:

- Network Construction

- Tool Used: SUMO’s NETEDIT.

- Process:

- A synthetic road network was designed to represent a simplified urban environment.

- The network consists of 13 nodes (intersections) and 19 edges (road segments).

- Each edge was assigned attributes such as length, maximum speed, and lane count to ensure realistic traffic flow.

- Rationale

- A moderately sized network balances computational efficiency with sufficient complexity to capture diverse EV movement patterns.

- The chosen topology allows multiple alternative paths between origin and destination nodes, enabling variability in route selection.

- Route Definition

- Configuration File: .rou.xml

- Process:

- All possible paths from Node 1 (origin) to Node 13 (destination) were enumerated.

- EV trips were scheduled with departure times distributed across the simulation horizon to mimic real-world stochastic travel behavior.

- Routing algorithms (e.g., Dijkstra’s shortest path) were applied to assign vehicles to edges based on travel cost.

- Rationale:

- Defining multiple routes ensures heterogeneity in traffic distribution.

- This step captures congestion effects and energy consumption variations across different paths.

- EV Modeling

  - EV type created using SUMO’s propulsion="electric" parameter.

  - Battery device enabled.

- Charging Station Deployment

  - One charging station added to every network node.

  - Attached via .add.xml.

- Simulation Execution

  - SUMO-GUI and SUMO-cli used.

  - SOC, speed, position logged at each timestep.

- Data Generation & Extraction

  - Travel-cost matrix constructed:

    - Distance

    - Travel time

    - Energy consumption

    - Charging demand

    - Node-level load

- Load Prediction Model

  - ML-based forecasting of charging load.

  - Inputs: speed, SOC, charging events, edge flows.

3.2 Data Collection

Data collected from SUMO simulation includes:

- Vehicle ID, route ID

- Time-step speed

- Battery SOC

- Charging events

- Node load per minute

- Energy consumed per route

3.3 Implementation

- SUMO v1.22

- Custom .net.xml from NETEDIT

- .rou.xml defining routes

- .add.xml with charging stations

- SUMO output modules for battery, speed, charging logs

- Python scripts for:

  - Data parsing

  - Travel-cost matrix

  - Load prediction

  - Visualizations

Chapter 4: Results and Discussion

4.1 Results

Key findings include:

- Node X and Y experience highest charging load.

- Energy consumption strongly correlates with route length and traffic congestion.

- Travel-cost matrix reveals variability across routes.

4.2 Discussion

- EV behavior influences load distribution non-linearly.

- Even low-traffic nodes can become load hotspots due to route clustering.

- Route optimization can reduce peak load .

4.3 Implications

- Power grids need adaptive response strategies.

- Charging stations should be prioritized at specific nodes.

- The methodology can be applied to real cities (Khulna, Jashore).

Chapter 5: Conclusion

5.1 Concluding Remarks

This research successfully integrates EV traffic simulation with power load prediction. The workflow efficiently models EV consumption and identifies high-load zones, providing a scalable approach for urban EV planning.

5.2 Limitations

- SUMO’s EV model simplifies battery chemistry.

- Real-world human behavior not fully represented.

- No stochastic traffic patterns included yet.

5.3 Recommendations

- Incorporating renewable energy charging simulations.

Moreover,

1. Integration of Real-World GPS and Traffic Data

2. Advanced Battery Degradation Modeling

3. Dynamic Charging Strategy Optimization

4. Renewable Energy and Smart Grid Integration

5. EV Type Diversification and Realistic Fleet Modeling

6. Predictive Load Forecasting Using Machine Learning

7. Multi-City or Nationwide Scaling

8. Economic and Policy Impact Analysis

9. Real-Time Simulation with Traffic Prediction

10. Human Behavior Modeling

References

- IEEE. (2021). Prediction of EV charging behavior using machine learning. IEEE Xplore. https://ieeexplore.ieee.org/document/9508419

- arXiv. (2025). Demand forecasting for electric vehicle charging stations using multivariate time-series analysis. arXiv. https://arxiv.org/abs/2502.16365

- IEEE. (2025). Machine learning based forecasting of EV charging load in a parking lot for optimal participation in frequency services. IEEE Xplore. https://ieeexplore.ieee.org/abstract/document/10854233

- arXiv. (2025). Dynamic load balancing for EV charging stations. arXiv. https://arxiv.org/abs/2503.06370

- IEEE. (2023). Electric vehicle charging demand prediction using machine learning algorithms. IEEE Xplore. https://ieeexplore.ieee.org/document/10677704

- E3S Web of Conferences. (2024). Machine learning models for predicting and managing electric vehicle load in smart grids. DOAJ. https://doaj.org/article/73c43328d7eb4e1e89f545c01751f968

- Elsevier. (2025). EV load forecasting using a refined CNN-LSTM-AM. ScienceDirect. https://github.com/gyboo/Refined-CNN-LSTM-AM

- Energies. (2019). Electric vehicle charging load forecasting: A comparative study of deep learning approaches. MDPI. https://www.mdpi.com/1996-1073/12/14/2692

- Energies. (2024). Electric vehicle charging load prediction considering spatio-temporal node importance information. MDPI. https://www.mdpi.com/1996-1073/17/19/4840

- Springer. (2025). A comprehensive benchmark of machine learning-based algorithms for medium-term EV charging demand prediction. The Journal of Supercomputing. https://link.springer.com/article/10.1007/s11227-025-06975-8

- World Electric Vehicle Journal. (2024). Electric vehicle charging load forecasting based on K-Means++-GRU-KSVR. MDPI. https://www.mdpi.com/2032-6653/15/12/582

- World Electric Vehicle Journal. (2025). Data-driven modeling of electric vehicle charging sessions Based on Machine Learning Techniques. MDPI. https://www.mdpi.com/2032-6653/16/2/107

- World Electric Vehicle Journal. (2025). Electric vehicle charging load forecasting method based on improved long short-term memory model with particle swarm optimization. MDPI. https://www.mdpi.com/2032-6653/16/3/150

- Energies. (2023). Multi-feature data fusion-based load forecasting of electric vehicle charging stations using a deep learning model. MDPI. https://www.mdpi.com/1996-1073/16/3/1309

- SSRN. (2024). Prediction of electric vehicle charging using machine learning. SSRN. https://www.bohrium.com/paper-details/prediction-of-electric-vehicle-charging-using-machine-learning/997218844353233591-16569

- Sustainability. (2023). Prediction of electric vehicles charging demand: A transformer-based deep learning approach. MDPI. https://www.mdpi.com/2071-1050/15/3/2105

- arXiv. (2024). Coherent hierarchical probabilistic forecasting of electric vehicle charging demand. arXiv. https://arxiv.org/abs/2411.00337

- Frontiers in Energy Research. (2023). Electric vehicle charging load prediction based on variational mode decomposition and Prophet-LSTM. Frontiers. https://www.frontiersin.org/journals/energy-research/articles/10.3389/fenrg.2023.1297849/full

- arXiv. (2024). Location-based probabilistic load forecasting of EV charging sites: Deep transfer learning with multi-quantile Temporal Convolutional Network. arXiv. https://arxiv.org/abs/2409.11862

- Scientific Reports. (2024). Machine learning optimization for hybrid electric vehicle charging. Nature. https://www.nature.com/articles/s41598-024-63775-5

- arXiv. (2024). Multiscale spatio-temporal enhanced short-term load forecasting of electric vehicle charging stations. arXiv. https://arxiv.org/abs/2405.19053

- Frontiers in Energy Research. (2024). Research on electric vehicle charging load prediction method based on spectral clustering. Frontiers. https://www.frontiersin.org/journals/energy-research/articles/10.3389/fenrg.2024.1294453/full

- IEEE. (2023). Electric vehicle charging load time-series prediction based on broad learning system. IEEE Xplore. https://ieeexplore.ieee.org/document/10128054

- IEEE. (2024). Prediction of electric vehicles charging load using long short-term memory. IEEE Xplore. https://ieeexplore.ieee.org/document/10426741

- Springer. (2024). Short-term aggregate electric vehicle charging load forecasting in diverse conditions with minimal data. Energy Systems. https://link.springer.com/article/10.1007/s12667-024-00682-8