Electric Vehicle Charging Station Load Prediction

Research Methodology Documentation

Khulna, Bangladesh Context

1. RESEARCH CONTEXT

1.1 Problem Statement

I am conducting research on battery-powered vehicle charging station load prediction in the context of Bangladesh, specifically focusing on Khulna region. Since there is currently no established electric vehicle charging infrastructure in Khulna, I cannot collect real-world charging data. Therefore, I have adopted a simulation-based approach using SUMO (Simulation of Urban MObility) to generate realistic charging load patterns.

1.2 Research Objective

The primary objective of my research is to develop a predictive model that can forecast charging station load demand based on historical usage patterns. This will help in:

- Planning future charging infrastructure deployment

- Optimizing station capacity allocation

- Managing grid load effectively

- Supporting policy decisions for EV adoption in Bangladesh

2. METHODOLOGY OVERVIEW

2.1 Simulation-Based Data Generation

Since real charging station data is unavailable in Khulna, I have created a comprehensive SUMO simulation that models:

Vehicle Types (Bangladesh-specific):

- Easy Bike

- E-Rickshaw

- E-Van

Charging Infrastructure:

- 10 charging stations (5 forward direction, 5 reverse direction)

- Station capacities: 4-5 vehicles per station

- Charging power: 20-25 kW per station

- Charging efficiency: 95%

Smart Charging Logic:

- Vehicles are rerouted to nearest charging station when SOC drops below 30%

- Target SOC after charging: 70%

- Dynamic charging duration calculation based on battery state and station power

2.2 Data Collection Approach

I have developed Python scripts using TraCI (Traffic Control Interface) to:

- Monitor vehicle battery states in real-time during simulation

- Track charging events (start time, duration, SOC changes)

- Record station-wise load demand every second

- Capture vehicle-station associations and occupancy

Output Data Files:

- load_demand_*.csv - Time-series power demand data

- station_snapshots_*.csv - Station utilization and occupancy

- charging_events_*.csv - Individual charging session details

- vehicle_tracking_*.csv - Vehicle movement and battery telemetry

3. PREDICTION METHODOLOGY

3.1 The Prediction Task

My prediction approach is structured as follows:

Training Phase: I use the first 80% of my simulation data (0-480 seconds) to train the prediction model. This data represents historical charging patterns.

Testing Phase: I use the remaining 20% of data (481-600 seconds) to validate the model's prediction accuracy. The model must predict future load demand based on patterns learned from historical data.

3.2 Why This Approach is Valid

This methodology is based on established practices in transportation and energy systems research:

- When real infrastructure doesn't exist, simulation provides realistic data

- Time-series splitting (80-20 train-test) validates predictive capability

- The model learns temporal patterns, station utilization trends, and vehicle behavior

3.3 Model Selection

I have chosen to implement Random Forest Regressor as my primary prediction model because:

- It handles non-linear relationships well

- Robust to outliers in charging patterns

- Provides feature importance analysis

- Computationally efficient for my dataset size

- Well-documented and widely used in energy forecasting

4. IMPLEMENTATION STEPS

Step 1: Simulation Execution

What I do:

Run: python charging_station_monitor.py

Duration: 600 seconds simulation time

Output: Multiple CSV files with complete charging data

Multiple Scenarios: I will run the simulation under different conditions:

- Scenario 1: Normal traffic (current configuration)

- Scenario 2: High traffic (increased vehicle flow rates by 50%)

- Scenario 3: Low initial SOC (vehicles start with 40% instead of current values)

This provides robust data covering various operational conditions.

Step 2: Data Preparation

What I do:

- Load the load_demand_*.csv file

- Engineer relevant features:

  - Temporal features: hour, minute, day of week

  - Rolling statistics: 5-min average, 15-min average, standard deviation

  - Lag features: previous 1, 5, 10, 30 timesteps

  - Peak/off-peak indicators

- Clean data (remove NaN values from lag features)

- Split data: 80% training (0-480s), 20% testing (481-600s)

Feature Set: My model uses these input features to predict total power demand:

- Time-based features (hour, minute cyclical encoding)

- Historical load (moving averages, lag values)

- Number of vehicles currently charging

- Station utilization percentage (if available)

- Peak hour indicator

Target Variable:

- Total power demand in kW at each timestep

Step 3: Model Training

What I do:

- Initialize Random Forest Regressor

- Scale features using StandardScaler

- Train model on 80% data

- Generate predictions for 20% test data

- Calculate performance metrics

Model Configuration:

Random Forest Parameters:

- n_estimators: 100 trees

- max_depth: 10

- random_state: 42 (for reproducibility)

Step 4: Model Evaluation

Metrics I Calculate:

- MAE (Mean Absolute Error): Average magnitude of prediction errors in kW

- RMSE (Root Mean Squared Error): Penalizes larger errors more heavily

- R² Score: Proportion of variance explained by the model (0-1, higher is better)

- MAPE (Mean Absolute Percentage Error): Error as percentage of actual values

What Good Performance Looks Like:

- RMSE < 3 kW (for my load range of 0-100 kW)

- R² > 0.85 (85% of variance explained)

- MAPE < 10% (less than 10% average error)

Step 5: Visualization and Analysis

Graphs I Generate:

- Prediction vs Actual Plot:

  - X-axis: Time (seconds)

  - Y-axis: Power demand (kW)

  - Two lines: Actual (from simulation), Predicted (from model)

  - Shows how well model tracks actual load

- Error Distribution Histogram:

  - Shows distribution of prediction errors

  - Should be centered around zero (unbiased predictions)

- Feature Importance Bar Chart:

  - Shows which features most influence predictions

  - Helps understand what drives charging load

- Station Utilization Heatmap:

  - Shows which stations are most utilized

  - When peak demand occurs

  - Informs infrastructure planning

Step 6: Results Interpretation

What I Report:

Quantitative Results: "The Random Forest model achieved an RMSE of [X] kW and R² of [Y], indicating that the model can predict charging load with [Y×100]% accuracy. The Mean Absolute Error of [Z] kW means predictions are typically within [Z] kW of actual demand."

Feature Analysis: "Feature importance analysis reveals that [top 3 features] are the primary drivers of charging demand, with [feature name] contributing [X]% to prediction accuracy."

Operational Insights: "Peak load occurs at [time], reaching [X] kW, with an average baseline load of [Y] kW. Station [ID] shows highest utilization at [Z]%, indicating potential need for capacity expansion."

5. VALIDATION OF SIMULATION-BASED APPROACH

5.1 Why Simulation Data is Acceptable

In my research context, using simulation data is not only acceptable but necessary because:

Practical Justification:

- No existing charging infrastructure in Khulna to collect real data from

- Simulation allows controlled experimentation with different scenarios

- Vehicle parameters are based on actual Bangladesh EV specifications

Academic Precedent: Many published transportation and energy research papers use simulation when:

- Infrastructure is in planning/design phase

- Real-world data collection is prohibitively expensive

- Studying future scenarios that don't yet exist

5.2 Ensuring Simulation Realism

I have ensured my simulation reflects Bangladesh reality through:

Realistic Vehicle Models:

- Battery capacities match actual Easy Bike/E-Rickshaw/E-Van specs

- Speed limits reflect typical urban Bangladesh traffic (7-8 m/s)

- Vehicle dimensions based on real measurements

Realistic Traffic Patterns:

- Flow rates calibrated to represent Khulna urban density

- Bidirectional traffic (forward and reverse routes)

- Multiple route choices mimicking real navigation behavior

Realistic Charging Behavior:

- 30% SOC threshold reflects real driver behavior (charge before empty)

- 70% target SOC reflects quick-charging preference (not full charge)

- Variable charging times based on actual battery physics

6. CONTRIBUTION TO BANGLADESH CONTEXT

6.1 Research Significance

My work provides:

For Policymakers:

- Data-driven capacity planning for future charging stations

- Understanding of peak demand periods for grid planning

- Cost-benefit analysis foundation for infrastructure investment

For Infrastructure Developers:

- Optimal station placement recommendations

- Capacity requirements per location

- Load management strategy insights

For Academic Community:

- First systematic study of EV charging load in Khulna context

- Validated prediction methodology applicable to similar developing regions

- Baseline data for future comparative studies

6.2 Practical Recommendations

Based on my prediction results, I will provide:

- Infrastructure Planning:

  - Recommended number and location of charging stations

  - Capacity requirements (kW) per station

  - Phased deployment strategy

- Load Management:

  - Time-of-use charging recommendations

  - Peak shaving strategies

  - Grid integration considerations

- Policy Implications:

  - Required grid capacity upgrades

  - Electricity tariff structure suggestions

  - Incentive mechanisms for off-peak charging

7. LIMITATIONS AND FUTURE WORK

7.1 Current Limitations

I acknowledge the following limitations:

Simulation Constraints:

- Simulation duration limited to 600 seconds (10 minutes)

- Fixed road network (doesn't capture entire Khulna city)

- Simplified driver behavior models

Data Constraints:

- No validation against real charging data (unavailable)

- Weather effects not modeled

- Seasonal variations not captured in single simulation

Model Constraints:

- Short-term prediction only (same-day)

- Doesn't account for special events (festivals, holidays)

- Grid capacity constraints not modeled

7.2 Future Research Directions

This work can be extended through:

- Longer Simulations:

  - 24-hour or multi-day simulations

  - Seasonal pattern analysis

  - Special event modeling (Eid, Durga Puja)

- Enhanced Models:

  - LSTM neural networks for longer-term prediction

  - Integration with weather data

  - Dynamic pricing optimization

- Validation Studies:

  - Once real infrastructure exists, validate against actual data

  - Calibrate simulation parameters based on real observations

  - Refine prediction models with real-world feedback

- Expanded Scope:

  - Full Khulna city network simulation

  - Integration with public transport planning

  - Vehicle-to-Grid (V2G) potential analysis

8. CONCLUSION

I have developed a comprehensive methodology for predicting electric vehicle charging station load demand in the absence of real-world infrastructure data. By leveraging SUMO simulation with Bangladesh-specific vehicle parameters and smart charging algorithms, I have generated realistic charging patterns. Using machine learning techniques (Random Forest), I can predict future load demand with high accuracy, providing valuable insights for infrastructure planning in Khulna and similar developing urban contexts.

This simulation-based prediction approach is scientifically valid, practically relevant, and provides a foundation for evidence-based policy decisions regarding EV infrastructure deployment in Bangladesh.

Document Version: 1.0
Last Updated: January 11, 2025
Research Context: MS Thesis - Battery-Powered Vehicle Charging Infrastructure
Location: Khulna, Bangladesh