COMPLETE CODE EXPLANATION

1. FILE HEADER & IMPORTS

"""

Enhanced Smart Charging Simulation with Queue Management

Features: Separate charging and waiting slots, intelligent queue management

"""

Purpose: Documentation string explaining what this module does - manages EV charging with separate charging/waiting areas.

import os

import sys

import traceback

from datetime import datetime

from collections import defaultdict, deque

Line-by-line:

- import os - Operating system functions (create folders, check files)

- import sys - System functions (exit program)

- import traceback - Error tracking (print detailed error messages)

- from datetime import datetime - Get current time for timestamps

- from collections import defaultdict, deque - Advanced data structures:

  - defaultdict - Dictionary with default values

  - deque - Double-ended queue (fast append/pop from both ends)

try:

import traci

print("✓ TraCI imported successfully")

except ImportError:

print("✗ ERROR: TraCI not found! Install: pip install traci")

sys.exit(1)

Purpose: Import TraCI (Traffic Control Interface) - the Python API for controlling SUMO

- try/except - Attempt to import, catch error if not installed

- sys.exit(1) - Exit program with error code 1 if TraCI missing

import pandas as pd

Purpose: Import pandas for data analysis (create CSV files from data)

2. CLASS DEFINITION & CONFIGURATION

class SmartChargingStationMonitor:

Purpose: Main class that manages the entire charging simulation

Configuration Constants

SOC_THRESHOLD = 30.0

SOC_TARGET = 70.0

MIN_CHARGE_TIME = 30

MAX_CHARGE_TIME = 300

Explanation:

- SOC_THRESHOLD = 30.0 - When to charge: If battery < 30%, vehicle needs charging

- SOC_TARGET = 70.0 - Charge until: Charge battery to 70%

- MIN_CHARGE_TIME = 30 - Minimum charging duration: At least 30 seconds

- MAX_CHARGE_TIME = 300 - Maximum charging duration: At most 300 seconds (5 minutes)

Why these values?

- 30% threshold prevents vehicles from running out of battery

- 70% target balances charging time vs. station availability

- Time limits prevent unrealistic charging scenarios

Charging Stations Dictionary

CHARGING_STATIONS = {

'pa_2': {

'charging_area': 'pa_2_charging',

'waiting_area': 'pa_2_waiting',

'lane': 'E0_0',

'edge': 'E0',

'power_kW': 20.0,

'efficiency': 0.95,

'charging_slots': 4,

'waiting_slots': 3

},

# ... more stations

}

Each station has:

- 'charging_area' - ID of parking area where vehicles charge (from XML file)

- 'waiting_area' - ID of parking area where vehicles wait when charging is full

- 'lane' - Specific lane ID in SUMO (e.g., "E0_0" = Edge0, Lane0)

- 'edge' - Road segment ID (e.g., "E0" or "-E0" for reverse direction)

- 'power_kW' - Charging power in kilowatts (20 kW or 25 kW)

- 'efficiency' - Charging efficiency 0.95 = 95% (5% energy loss)

- 'charging_slots' - How many vehicles can charge simultaneously (4-5)

- 'waiting_slots' - How many vehicles can wait in queue (2-3)

Why separate forward/reverse stations?

- Roads are bidirectional in SUMO

- "-E0" = reverse direction of edge E0

- Vehicles traveling opposite directions need different stations

3. INITIALIZATION METHOD

def __init__(self, sumocfg='Test1.sumocfg', output_folder='simulation_outputs'):

Purpose: Constructor - runs when creating a SmartChargingStationMonitor object

self.sumocfg = sumocfg

self.output_folder = output_folder

os.makedirs(output_folder, exist_ok=True)

Explanation:

- self.sumocfg - Store path to SUMO configuration file

- self.output_folder - Where to save CSV results

- os.makedirs(..., exist_ok=True) - Create folder if it doesn't exist; don't error if it does

Data Storage Lists

self.vehicle_data = []

self.charging_events = []

self.low_soc_alerts = []

self.load_demand_data = []

self.vehicle_states = {}

self.initial_soc = {}

Purpose: Initialize empty containers for collected data

- vehicle_data - List of dictionaries: every vehicle's position/SOC/status each timestep

- charging_events - List: record of completed charging sessions

- low_soc_alerts - List: log when vehicles need charging

- load_demand_data - List: power consumption over time

- vehicle_states - Dictionary: current state of each vehicle (charging status, route, etc.)

- initial_soc - Dictionary: starting battery level of each vehicle

Station Tracking Data Structures

self.station_charging_slots = {}  # {station_id: set(vehicle_ids)}

self.station_waiting_queue = {}   # {station_id: deque(vehicle_ids)} - FIFO queue

self.currently_charging = {}      # {vehicle_id: station_id}

self.currently_waiting = {}       # {vehicle_id: station_id}

Detailed Explanation:

self.station_charging_slots - Which vehicles are charging at each station

- Example: {'pa_2': {'veh1', 'veh2'}, 'pa_3': {'veh5'}}

- Uses set() for fast add/remove operations

self.station_waiting_queue - Queue of vehicles waiting at each station

- Example: {'pa_2': deque(['veh3', 'veh4', 'veh6'])}

- Uses deque() for FIFO (First-In-First-Out) queue

- Vehicle at front of queue gets promoted first

self.currently_charging - Reverse lookup: which station is vehicle at?

- Example: {'veh1': 'pa_2', 'veh2': 'pa_2'}

- Allows quick check: "Is veh1 charging? If yes, where?"

self.currently_waiting - Which vehicles are in waiting areas

- Example: {'veh3': 'pa_2', 'veh4': 'pa_2'}

Initialize Station Containers

for station_id in self.CHARGING_STATIONS.keys():

self.station_charging_slots[station_id] = set()

self.station_waiting_queue[station_id] = deque()

Purpose: Create empty set/deque for each station

- Before: self.station_charging_slots = {}

- After: self.station_charging_slots = {'pa_2': set(), 'pa_2_rev': set(), ...}

Monitoring Data Structures

self.station_snapshots = []

self.station_vehicle_history = defaultdict(list)

self.queue_events = []

Explanation:

- station_snapshots - Periodic snapshots of station utilization

- station_vehicle_history - defaultdict automatically creates empty list if key doesn't exist

- queue_events - Log of vehicles joining/leaving queues

Initialization Summary Print

print("✓ Enhanced Charging Station Monitor with Queue Management Initialized")

print(f"  SOC: {self.SOC_THRESHOLD}% → {self.SOC_TARGET}%")

total_charging = sum(s['charging_slots'] for s in self.CHARGING_STATIONS.values())

total_waiting = sum(s['waiting_slots'] for s in self.CHARGING_STATIONS.values())

print(f"  Stations: {len(self.CHARGING_STATIONS)} ({total_charging} charging slots + {total_waiting} waiting slots)")

Purpose: Print configuration summary

- Count total charging slots across all stations

- Count total waiting slots

- Example output: "Stations: 10 (43 charging slots + 26 waiting slots)"

4. BATTERY INFORMATION METHOD

def get_battery_info(self, vid):

Purpose: Get battery status for a vehicle

try:

try:

charge = traci.vehicle.getParameter(vid, "device.battery.chargeLevel")

capacity = traci.vehicle.getParameter(vid, "device.battery.capacity")

energy = traci.vehicle.getParameter(vid, "device.battery.totalEnergyConsumed")

Explanation:

- traci.vehicle.getParameter() - Get vehicle parameter from SUMO

- "device.battery.chargeLevel" - Current battery charge in Wh (Watt-hours)

- "device.battery.capacity" - Maximum battery capacity in Wh

- "device.battery.totalEnergyConsumed" - Total energy used since start

Nested try/except - Different SUMO versions use different parameter names

except:

charge = traci.vehicle.getParameter(vid, "actualBatteryCapacity") or \

traci.vehicle.getParameter(vid, "device.battery.actualBatteryCapacity")

capacity = traci.vehicle.getParameter(vid, "maximumBatteryCapacity") or \

traci.vehicle.getParameter(vid, "device.battery.maximumBatteryCapacity")

Fallback: Try alternative parameter names if first attempt fails

c = float(charge) if charge else 0.0

cap = float(capacity) if capacity else 0.0

e = float(energy) if energy else 0.0

soc = (c / cap * 100.0) if cap > 0 else 0.0

Calculations:

- Convert strings to floats, default to 0 if None

- SOC (State of Charge) = (current_charge / max_capacity) × 100

- Example: (2700 Wh / 7200 Wh) × 100 = 37.5%

return {'charge_Wh': c, 'capacity_Wh': cap, 'soc_percent': soc, 'energy_consumed_Wh': e}

except:

return None

Return: Dictionary with battery info, or None if error

5. STATION DETECTION METHOD

def get_charging_station_id(self, vid):

"""Detect if vehicle is at a charging station (charging or waiting area)"""

if vid in self.currently_charging:

return self.currently_charging[vid]

if vid in self.currently_waiting:

return self.currently_waiting[vid]

Fast Lookup: If already tracked, return station ID immediately

- Example: If self.currently_charging = {'veh1': 'pa_2'}, return 'pa_2'

try:

stops = traci.vehicle.getStops(vid)

if stops:

for stop in stops:

if hasattr(stop, 'stoppingPlaceID'):

place_id = stop.stoppingPlaceID

Explanation:

- traci.vehicle.getStops(vid) - Get list of scheduled stops

- Each stop has a stoppingPlaceID (parking area ID)

- Loop through stops to find parking area

for station_id, info in self.CHARGING_STATIONS.items():

if place_id == info.get('charging_area') or place_id == info.get('waiting_area'):

speed = traci.vehicle.getSpeed(vid)

if speed < 0.5:

return station_id

Logic:

- Check if place_id matches any charging or waiting area

- Verify vehicle is stopped (speed < 0.5 m/s)

- Return station ID if match found

Why check speed?

- Vehicle might be driving past parking area

- Only consider it "at station" if actually stopped

6. STATION AVAILABILITY METHOD

def is_station_available(self, station_id, check_waiting=True):

"""

Check if station has availability

Returns: 'charging' if charging slot available, 'waiting' if only waiting slot available, None if full

"""

station_info = self.CHARGING_STATIONS[station_id]

charging_occupied = len(self.station_charging_slots[station_id])

waiting_occupied = len(self.station_waiting_queue[station_id])

Count Occupancy:

- How many vehicles currently charging?

- How many vehicles in waiting queue?

if charging_occupied < station_info['charging_slots']:

return 'charging'

Check Charging Slots:

- Example: If 3 vehicles charging and capacity is 4 → return 'charging'

if check_waiting and waiting_occupied < station_info['waiting_slots']:

return 'waiting'

Check Waiting Slots:

- Only check if check_waiting=True

- Example: If 2 vehicles waiting and capacity is 3 → return 'waiting'

return None

Station Full: No slots available

7. NEAREST STATION FINDER

def find_nearest_reachable_station(self, vid):

Purpose: Find nearest available station using intelligent routing rules

try:

cur_edge = traci.vehicle.getRoadID(vid)

if not cur_edge or cur_edge.startswith(':'):

return None

except:

return None

Get Current Location:

- traci.vehicle.getRoadID() - Which road is vehicle on?

- Skip if on junction (starts with ':')

candidates = []

for sid, info in self.CHARGING_STATIONS.items():

try:

route = traci.simulation.findRoute(cur_edge, info['edge'])

if route and route.length > 0:

candidates.append((route.length, sid, info, route))

except:

continue

Build Candidate List:

- traci.simulation.findRoute() - Find shortest path to station

- Store: (distance, station_id, station_info, route_object)

- Example: [(850.5, 'pa_2', {...}, route_obj), (1200.3, 'pa_3', {...}, route_obj)]

if not candidates:

return None

candidates.sort(key=lambda x: x[0])

Sort by Distance: Closest station first

Intelligent Routing Rules

# RULE 1: Nearest → CHARGING ONLY

dist1, sid1, info1, route1 = candidates[0]

if self.is_station_available(sid1, check_waiting=False) == 'charging':

return {

'id': sid1,

'info': info1,

'route_to': route1,

'availability': 'charging',

'total_length': dist1

}

Rule 1: If nearest station has charging slot available → go there

- check_waiting=False - Don't accept waiting slot at nearest station

- Why? Prefer going slightly farther to charge immediately

# RULE 2-5: Second nearest

if len(candidates) < 2:

return None

dist2, sid2, info2, route2 = candidates[1]

availability2 = self.is_station_available(sid2, check_waiting=True)

Rule 2: Check second-nearest station

# Rule 3

if availability2 == 'charging':

return {

'id': sid2,

'info': info2,

'route_to': route2,

'availability': 'charging',

'total_length': dist2

}

Rule 3: If second-nearest has charging → go there

- Reason: Nearest is full but second-nearest can charge immediately

# Rule 4

if availability2 == 'waiting':

return {

'id': sid2,

'info': info2,

'route_to': route2,

'availability': 'waiting',

'total_length': dist2

}

Rule 4: If second-nearest has waiting slot → go there

- Reason: Better to wait at second station than nothing

# Rule 5: Do NOT route anywhere

return None

Rule 5: Both nearest stations full → don't route (vehicle continues driving)

8. CHARGE TIME CALCULATION

def calculate_charge_time(self, batt, station):

Purpose: Calculate how long vehicle needs to charge

soc = batt.get('soc_percent', 0.0)

cap = batt.get('capacity_Wh', 0.0)

needed = max(0.0, (self.SOC_TARGET - soc) / 100.0 * cap)

Energy Needed:

- Current SOC: 35%

- Target SOC: 70%

- Capacity: 7200 Wh

- Calculation: (70 - 35) / 100 × 7200 = 0.35 × 7200 = 2520 Wh needed

power = station.get('power_kW', 20.0) * 1000.0

eff = station.get('efficiency', 0.95)

Station Power:

- Convert kW to W: 20 kW × 1000 = 20,000 W

- Efficiency: 95% (5% loss)

if power * eff > 0 and needed > 0:

time = (needed / (power * eff)) * 3600.0

else:

time = self.MIN_CHARGE_TIME

Time Calculation:

- Effective Power: 20,000 W × 0.95 = 19,000 W

- Time = (2520 Wh / 19,000 W) × 3600 = 0.1326 hours × 3600 = 477 seconds

Why × 3600?

- Power is in Watts (Joules/second)

- Need time in seconds

- Wh / W gives hours → multiply by 3600 to get seconds

return max(self.MIN_CHARGE_TIME, min(self.MAX_CHARGE_TIME, time))

Apply Limits:

- If calculated time < 30s → use 30s

- If calculated time > 300s → use 300s

- Otherwise use calculated time

9. ROUTING AND SCHEDULING

def reroute_and_schedule_charge(self, vid, choice, batt):

"""Route vehicle to station and assign to charging or waiting slot"""

try:

sid = choice['id']

info = choice['info']

route_to = choice.get('route_to')

availability = choice.get('availability')

charge_time = self.calculate_charge_time(batt, info)

Extract Information:

- Station ID, station info, route, availability type

- Calculate how long to charge

# Build route

edges = list(route_to.edges) if route_to and hasattr(route_to, 'edges') else []

if not edges:

return False

Get Route Edges:

- SUMO routes are lists of edge IDs

- Example: ['E0', 'E1', 'E5']

# Add return route

try:

vroute = traci.vehicle.getRoute(vid)

dest = vroute[-1] if vroute else None

if dest:

rb = traci.simulation.findRoute(info['edge'], dest)

if rb and hasattr(rb, 'edges'):

back = rb.edges

if back and edges and back[0] == edges[-1]:

edges.extend(back[1:])

else:

edges.extend(back)

except:

pass

Build Complete Route:

- Get vehicle's original destination

- Find route from station back to destination

- Combine: [current_pos → station → original_destination]

Why check back[0] == edges[-1]?

- Avoid duplicate edge in route

- Example: ['E0', 'E1'] + ['E1', 'E5'] → remove duplicate E1

# Clean route

cleaned = []

for e in edges:

if not cleaned or cleaned[-1] != e:

cleaned.append(e)

Remove Consecutive Duplicates:

- ['E0', 'E1', 'E1', 'E5'] → ['E0', 'E1', 'E5']

# Set route and stop

if availability == 'charging':

parking_area = info['charging_area']

elif availability == 'waiting':

parking_area = info['waiting_area']

else:

parking_area = info.get('charging_area', sid)

Choose Parking Area:

- If charging slot available → use charging area

- If only waiting slot → use waiting area

traci.vehicle.setRoute(vid, cleaned)

traci.vehicle.setParkingAreaStop(vid, parking_area, duration=charge_time)

Send Commands to SUMO:

- setRoute() - Tell vehicle which edges to drive

- setParkingAreaStop() - Tell vehicle to stop at parking area for X seconds

# Update vehicle state

if vid not in self.vehicle_states:

self.vehicle_states[vid] = {}

self.vehicle_states[vid].update({

'routed_to': sid,

'routed_time': traci.simulation.getTime(),

'scheduled_charge_time': charge_time,

'charge_scheduled': True,

'expected_slot': availability

})

Track Vehicle State:

- Remember which station vehicle is going to

- When routing happened

- How long it should charge

- Mark as "charging scheduled" to prevent re-routing

print(f"🔌 {vid} → {sid} [{availability}] | {charge_time:.0f}s | {choice.get('total_length',0):.1f}m")

return True

except Exception as e:

print(f"⚠ Route failed: {e}")

return False

Logging and Error Handling:

- Print success message

- Return True if successful, False if failed

10. SLOT ASSIGNMENT

def assign_vehicle_to_slot(self, vid, station_id, t):

"""Assign vehicle to charging or waiting parking area based on availability"""

availability = self.is_station_available(station_id, check_waiting=True)

station_info = self.CHARGING_STATIONS[station_id]

Check Availability: Is there space at this station?

Charging Slot Assignment

if availability == 'charging':

charging_area_id = station_info['charging_area']

self.station_charging_slots[station_id].add(vid)

self.currently_charging[vid] = station_id

Add to Tracking:

- Add vehicle to station's charging set

- Record vehicle is charging at this station

if vid not in self.vehicle_states:

self.vehicle_states[vid] = {}

batt = self.get_battery_info(vid)

soc = batt['soc_percent'] if batt else 0.0

Get Battery Info: Need SOC to track charging progress

self.vehicle_states[vid].update({

'charging_start': t,

'soc_start': soc,

'charging_station_active': station_id,

'slot_type': 'charging',

'parking_area': charging_area_id

})

Record Charging Start:

- When charging started (current timestep)

- SOC at start (to calculate gain later)

- Which station

- What type of slot

print(f"🔋 {vid} @ {station_id} [CHARGING AREA] | {soc:.1f}%")

return True

Log and Return: Print message, return success

Waiting Slot Assignment

elif availability == 'waiting':

waiting_area_id = station_info['waiting_area']

self.station_waiting_queue[station_id].append(vid)

self.currently_waiting[vid] = station_id

Add to Queue:

- Append to end of queue (FIFO)

- Mark as waiting at this station

self.vehicle_states[vid].update({

'waiting_start': t,

'waiting_station': station_id,

'slot_type': 'waiting',

'parking_area': waiting_area_id

})

queue_pos = len(self.station_waiting_queue[station_id])

print(f"⏳ {vid} @ {station_id} [WAITING AREA] | Queue pos: {queue_pos}")

Track Waiting:

- When started waiting

- Queue position (1st, 2nd, 3rd, etc.)

self.queue_events.append({

'timestep_sec': t,

'vehicle_id': vid,

'station_id': station_id,

'event': 'joined_queue',

'queue_position': queue_pos

})

return True

Log Queue Event: For analysis later

Station Full

else:

print(f"❌ {vid} @ {station_id} - STATION FULL!")

return False

No Space: Should not happen if routing logic works correctly

11. QUEUE PROMOTION

def promote_waiting_vehicles(self, station_id, t):

"""Move vehicles from waiting area to charging area when slots available"""

Purpose: When charging slot opens, move waiting vehicle to charge

promoted_count = 0

station_info = self.CHARGING_STATIONS[station_id]

while self.station_waiting_queue[station_id]:

Loop: Continue while there are vehicles waiting

if self.is_station_available(station_id, check_waiting=False) != 'charging':

break

Check Availability: Stop if no charging slots available

vid = self.station_waiting_queue[station_id].popleft()

Get Next Vehicle: Remove from front of queue (FIFO)

try:

if vid not in traci.vehicle.getIDList():

self.currently_waiting.pop(vid, None)

continue

except:

self.currently_waiting.pop(vid, None)

continue

Verify Exists: Vehicle might have left simulation

self.currently_waiting.pop(vid, None)

batt = self.get_battery_info(vid)

if not batt:

continue

charge_time = self.calculate_charge_time(batt, station_info)

Prepare Promotion:

- Remove from waiting tracking

- Get battery info

- Calculate charge time

try:

charging_area_id = station_info['charging_area']

traci.vehicle.resume(vid)

traci.vehicle.setParkingAreaStop(vid, charging_area_id, duration=charge_time)

Move Vehicle:

- resume() - Release from current stop (waiting area)

- setParkingAreaStop() - Stop at charging area instead

except Exception as e:

print(f"⚠ Could not promote {vid} to charging area: {e}")

continue

Error Handling: Skip if command fails

self.station_charging_slots[station_id].add(vid)

self.currently_charging[vid] = station_id

Update Tracking:

- Add to charging slots

- Mark as charging

vs = self.vehicle_states.get(vid, {})

wait_start = vs.pop('waiting_start', t)

wait_time = t - wait_start

soc = batt['soc_percent']

Calculate Wait Time:

- When did waiting start?

- How long waited? (current_time - start_time)

vs.update({

'charging_start': t,

'soc_start': soc,

'charging_station_active': station_id,

'slot_type': 'charging',

'wait_time_sec': wait_time,

'parking_area': charging_area_id

})

Update State: Now charging (not waiting)

print(f"⬆️ {vid} @ {station_id} [WAITING→CHARGING] | Waited {wait_time:.0f}s | {soc:.1f}%")

self.queue_events.append({

'timestep_sec': t,

'vehicle_id': vid,

'station_id': station_id,

'event': 'promoted_to_charging',

'wait_time_sec': round(wait_time, 1)

})

promoted_count += 1

Log Promotion: Record event, increment counter

return promoted_count

Return: How many vehicles promoted

12. MONITORING METHOD

def monitor_charging(self, t):

"""Monitor charging and waiting vehicles, manage queue promotions"""

Purpose: Main monitoring loop - check all vehicles every timestep

try:

vids = set(traci.vehicle.getIDList())

except:

return

Get All Vehicles: Set of currently active vehicle IDs

for vid in list(self.vehicle_states.keys()):

if vid not in vids:

self.cleanup_departed_vehicle(vid)

Clean Departed: Remove tracking for vehicles that left simulation

for vid in vids:

cs = self.get_charging_station_id(vid)

Check Each Vehicle: Is it at a station?

if cs and cs in self.CHARGING_STATIONS:

vs = self.vehicle_states.get(vid, {})

if vid not in self.currently_charging and vid not in self.currently_waiting:

self.assign_vehicle_to_slot(vid, cs, t)

New Arrival:

- Vehicle is at station

- Not yet tracked as charging/waiting

- → Assign to slot

else:

if vid in self.currently_charging:

self.handle_charging_complete(vid, t)

elif vid in self.currently_waiting:

self.handle_waiting_left(vid, t)

Departure:

- Vehicle was charging but now left → handle completion

- Vehicle was waiting but now left → handle queue exit

for station_id in self.CHARGING_STATIONS.keys():

if self.station_waiting_queue[station_id]:

self.promote_

waiting_vehicles(station_id, t)

**Check Promotions:** For each station with waiting vehicles, try to promote

---

## **13. CLEANUP METHOD**

```python

def cleanup_departed_vehicle(self, vid):

"""Clean up tracking for departed vehicle"""

vs = self.vehicle_states.get(vid, {})

station = vs.get('routed_to') or vs.get('charging_station_active') or vs.get('waiting_station')

Find Station: Where was this vehicle?

if vid in self.currently_charging:

if station and station in self.station_charging_slots:

self.station_charging_slots[station].discard(vid)

self.currently_charging.pop(vid, None)

Remove from Charging:

- Remove from station's slot set

- Remove from currently_charging dict

if vid in self.currently_waiting:

if station and station in self.station_waiting_queue:

try:

self.station_waiting_queue[station].remove(vid)

except ValueError:

pass

self.currently_waiting.pop(vid, None)

Remove from Waiting:

- Remove from queue

- Remove from currently_waiting dict

self.vehicle_states.pop(vid, None)

Clear State: Delete all tracking data

14. CHARGING COMPLETE HANDLER

def handle_charging_complete(self, vid, t):

"""Handle vehicle leaving charging slot"""

station = self.currently_charging[vid]

vs = self.vehicle_states.get(vid, {})

start = vs.get('charging_start', t)

soc0 = vs.get('soc_start', 0)

wait_time = vs.get('wait_time_sec', 0)

Get Info:

- Which station?

- When did charging start?

- What was starting SOC?

- How long waited before charging?

batt = self.get_battery_info(vid)

if not batt:

self.station_charging_slots[station].discard(vid)

del self.currently_charging[vid]

vs.pop('charging_start', None)

return

Safety Check: If can't get battery info, just clean up and exit

dur = t - start

Duration: How long charged? (current_time - start_time)

self.charging_events.append({

'vehicle_id': vid,

'vehicle_type': traci.vehicle.getTypeID(vid),

'charging_station': station,

'start_time_sec': start,

'end_time_sec': t,

'duration_sec': round(dur, 1),

'wait_time_sec': round(wait_time, 1),

'soc_at_start_percent': round(soc0, 2),

'soc_at_end_percent': round(batt['soc_percent'], 2),

'soc_gained_percent': round(batt['soc_percent'] - soc0, 2)

})

Log Event: Complete charging record for analysis

self.station_charging_slots[station].discard(vid)

del self.currently_charging[vid]

vs.pop('charging_start', None)

vs.pop('soc_start', None)

vs.pop('charging_station_active', None)

vs.pop('routed_to', None)

vs.pop('charge_scheduled', None)

vs.pop('slot_type', None)

vs.pop('wait_time_sec', None)

Clean Up: Remove from tracking, clear state variables

print(f"✅ {vid} @ {station} | {dur:.0f}s charge | {soc0:.1f}%→{batt['soc_percent']:.1f}% (+{batt['soc_percent']-soc0:.1f}%)")

Log: Print completion message

15. WAITING LEFT HANDLER

def handle_waiting_left(self, vid, t):

"""Handle vehicle leaving waiting queue"""

Purpose: Vehicle left waiting area without charging

station = self.currently_waiting[vid]

vs = self.vehicle_states.get(vid, {})

wait_start = vs.get('waiting_start', t)

wait_time = t - wait_start

Calculate Wait: How long was vehicle waiting?

self.queue_events.append({

'timestep_sec': t,

'vehicle_id': vid,

'station_id': station,

'event': 'left_queue',

'wait_time_sec': round(wait_time, 1)

})

Log Event: For analysis - vehicle gave up waiting

try:

self.station_waiting_queue[station].remove(vid)

except ValueError:

pass

del self.currently_waiting[vid]

vs.pop('waiting_start', None)

vs.pop('waiting_station', None)

vs.pop('slot_type', None)

Clean Up: Remove from queue and tracking

16. STATION SNAPSHOT

def capture_station_snapshot(self, t):

"""Capture detailed snapshot of all stations"""

Purpose: Record current state of all stations for analysis

for station_id, station_info in self.CHARGING_STATIONS.items():

Loop Through Stations: Process each one

Charging Vehicles

charging_vids = list(self.station_charging_slots[station_id])

charging_times = []

total_power_kW = 0.0

for vid in charging_vids:

vs = self.vehicle_states.get(vid, {})

charge_start = vs.get('charging_start', t)

duration = t - charge_start

charging_times.append(round(duration, 1))

total_power_kW += station_info['power_kW']

For Each Charging Vehicle:

- How long has it been charging?

- Add up total power consumption

batt = self.get_battery_info(vid)

soc = batt['soc_percent'] if batt else 0.0

self.station_vehicle_history[station_id].append({

'timestep_sec': t,

'vehicle_id': vid,

'vehicle_type': traci.vehicle.getTypeID(vid),

'slot_type': 'charging',

'charging_duration_sec': round(duration, 1),

'current_soc_percent': round(soc, 2),

'power_consumption_kW': station_info['power_kW']

})

Record History: Detailed log for each vehicle

Waiting Vehicles

waiting_vids = list(self.station_waiting_queue[station_id])

waiting_times = []

for vid in waiting_vids:

vs = self.vehicle_states.get(vid, {})

wait_start = vs.get('waiting_start', t)

duration = t - wait_start

waiting_times.append(round(duration, 1))

self.station_vehicle_history[station_id].append({

'timestep_sec': t,

'vehicle_id': vid,

'vehicle_type': traci.vehicle.getTypeID(vid),

'slot_type': 'waiting',

'waiting_duration_sec': round(duration, 1),

'current_soc_percent': 0,

'power_consumption_kW': 0

})

Track Waiting: Similar but no power consumption

Create Snapshot

snapshot = {

'timestep_sec': t,

'station_id': station_id,

'station_edge': station_info['edge'].replace('-', 'NEG_') if station_info['edge'].startswith('-') else station_info['edge'],

'charging_capacity': station_info['charging_slots'],

'waiting_capacity': station_info['waiting_slots'],

'vehicles_charging': len(charging_vids),

'vehicles_waiting': len(waiting_vids),

'charging_vehicle_ids': ','.join(charging_vids) if charging_vids else 'NONE',

'waiting_vehicle_ids': ','.join(waiting_vids) if waiting_vids else 'NONE',

'charging_times_sec': ','.join(map(str, charging_times)) if charging_times else 'NONE',

'waiting_times_sec': ','.join(map(str, waiting_times)) if waiting_times else 'NONE',

'total_power_load_kW': round(total_power_kW, 3),

'charging_utilization_percent': round((len(charging_vids) / station_info['charging_slots']) * 100, 1),

'waiting_utilization_percent': round((len(waiting_vids) / station_info['waiting_slots']) * 100, 1) if station_info['waiting_slots'] > 0 else 0,

'available_charging_slots': station_info['charging_slots'] - len(charging_vids),

'available_waiting_slots': station_info['waiting_slots'] - len(waiting_vids)

}

Complete Station State:

- How many vehicles charging/waiting

- Who they are

- How long they've been there

- Power consumption

- Utilization percentage

- Available slots

self.station_snapshots.append(snapshot)

Save Snapshot: Add to list for later export

17. LOAD CALCULATION

def calculate_load(self, t):

"""Calculate system-wide load"""

Purpose: Total power consumption across all stations

loads = {s: {'vehicles': 0, 'power_kW': 0.0} for s in self.CHARGING_STATIONS}

total = 0.0

Initialize: Empty dict for each station

for vid, station_id in self.currently_charging.items():

if station_id in self.CHARGING_STATIONS:

power = self.CHARGING_STATIONS[station_id]['power_kW']

loads[station_id]['vehicles'] += 1

loads[station_id]['power_kW'] += power

total += power

Sum Power:

- For each charging vehicle

- Add power to station total

- Add to overall total

rec = {

'timestep_sec': t,

'total_vehicles_charging': sum(l['vehicles'] for l in loads.values()),

'total_vehicles_waiting': sum(len(q) for q in self.station_waiting_queue.values()),

'total_power_demand_kW': round(total, 3),

'total_power_demand_MW': round(total / 1000, 6)

}

Create Record:

- Total vehicles charging

- Total vehicles waiting

- Power in kW and MW

for s, d in loads.items():

rec[f'{s}_vehicles'] = d['vehicles']

rec[f'{s}_power_kW'] = round(d['power_kW'], 3)

Per-Station Data: Add columns for each station

self.load_demand_data.append(rec)

Save Record: Add to list

18. DATA COLLECTION

def collect_data(self, t):

Purpose: Main data collection - called every second

self.monitor_charging(t)

self.capture_station_snapshot(t)

self.calculate_load(t)

Station Data: Update monitoring, snapshots, load

try:

vids = traci.vehicle.getIDList()

except:

return

for v in vids:

Vehicle Loop: Process each vehicle

try:

b = self.get_battery_info(v)

if not b:

continue

if v not in self.initial_soc:

self.initial_soc[v] = b['soc_percent']

Battery Info:

- Get current battery state

- Record initial SOC (first time seeing vehicle)

vs = self.vehicle_states.get(v, {})

if b['soc_percent'] < self.SOC_THRESHOLD and not vs.get('charge_scheduled'):

choice = self.find_nearest_reachable_station(v)

if choice:

self.low_soc_alerts.append({

'timestep_sec': t,

'vehicle_id': v,

'vehicle_type': traci.vehicle.getTypeID(v),

'current_soc_percent': round(b['soc_percent'], 2),

'nearest_station': choice['id'],

'route_length_m': round(choice.get('total_length', 0), 2),

'slot_availability': choice.get('availability', 'unknown')

})

self.reroute_and_schedule_charge(v, choice, b)

Charging Logic:

- If SOC < 30% AND not already scheduled

- Find nearest station

- Log alert

- Route vehicle

try:

pos = traci.vehicle.getPosition(v)

spd = traci.vehicle.getSpeed(v)

dist = traci.vehicle.getDistance(v)

lane = traci.vehicle.getLaneID(v)

road = traci.vehicle.getRoadID(v)

except:

pos, spd, dist, lane, road = (0,0), 0, 0, "UNK", "UNK"

Get Position Data: Location, speed, distance traveled

status = 'driving'

if v in self.currently_charging:

status = 'charging'

elif v in self.currently_waiting:

status = 'waiting'

Determine Status: What is vehicle doing?

self.vehicle_data.append({

'timestep_sec': t,

'vehicle_id': v,

'vehicle_type': traci.vehicle.getTypeID(v),

'status': status,

'soc_percent': round(b['soc_percent'], 2),

'initial_soc_percent': round(self.initial_soc.get(v, b['soc_percent']), 2),

'soc_drop_percent': round(self.initial_soc.get(v, b['soc_percent']) - b['soc_percent'], 2),

'energy_consumed_Wh': round(b['energy_consumed_Wh'], 2),

'lane': str(lane),

'road_id': str(road),

'x_position': round(pos[0], 2),

'y_position': round(pos[1], 2),

'speed_ms': round(spd, 2),

'distance_m': round(dist, 2)

})

Record Vehicle Data: Complete snapshot of vehicle state

19. SIMULATION RUNNER

def run_simulation(self, gui=True, max_time=600):

Purpose: Run SUMO simulation

print(f"\n{'='*70}\nENHANCED QUEUE MANAGEMENT SIMULATION\n{'='*70}")

cmd = ["sumo-gui" if gui else "sumo", "-c", self.sumocfg, "--start", "--quit-on-end"]

Build Command:

- Use sumo-gui (visual) or sumo (headless)

- -c - configuration file

- --start - start immediately

- --quit-on-end - close when done

try:

traci.start(cmd)

print("✓ Started\n")

except Exception as e:

print(f"✗ Failed: {e}")

return False

Start SUMO: Launch simulation

try:

t = 0

while traci.simulation.getMinExpectedNumber() > 0 and t < max_time:

traci.simulationStep()

t = traci.simulation.getTime()

Simulation Loop:

- Continue while vehicles remain

- Stop at max_time

- simulationStep() - advance 1 timestep (0.1s by default)

if int(t) == t:

self.collect_data(t)

Collect Data: Every integer second (1.0, 2.0, 3.0...)

if int(t) % 30 == 0 and t > 0:

active = len(traci.vehicle.getIDList())

chrg = len(self.currently_charging)

wait = sum(len(q) for q in self.station_waiting_queue.values())

load = self.load_demand_data[-1]['total_power_demand_kW'] if self.load_demand_data else 0

print(f"⏱️ {int(t)}s | Active:{active} Charging:{chrg} Waiting:{wait} Load:{load:.1f}kW")

Progress Updates: Every 30 seconds, print status

traci.close()

print(f"\n✓ Done at {t}s")

return True

except Exception as e:

print(f"✗ Error: {e}")

traceback.print_exc()

try:

traci.close()

except:

pass

return False

Cleanup: Close SUMO, handle errors

20. RESULTS EXPORT

def export_results(self):

Purpose: Save all collected data to CSV files

print(f"\n{'='*70}\nEXPORTING RESULTS\n{'='*70}")

ts = datetime.now().strftime("%Y%m%d_%H%M%S")

Timestamp: Create unique filename (YYYYMMDD_HHMMSS)

Queue Events

if self.queue_events:

df = pd.DataFrame(self.queue_events)

f = os.path.join(self.output_folder, f'queue_events_{ts}.csv')

df.to_csv(f, index=False)

print(f"✓ Queue Events: {f} ({len(df):,} events)")

joined = df[df['event'] == 'joined_queue']

promoted = df[df['event'] == 'promoted_to_charging']

if len(promoted) > 0:

avg_wait = promoted['wait_time_sec'].mean()

max_wait = promoted['wait_time_sec'].max()

print(f"  Avg Wait: {avg_wait:.1f}s | Max Wait: {max_wait:.1f}s")

Export Queue Data:

- Convert list of dicts to DataFrame

- Save as CSV

- Calculate statistics (average/max wait time)

Station Snapshots

if self.station_snapshots:

df = pd.DataFrame(self.station_snapshots)

f = os.path.join(self.output_folder, f'station_snapshots_{ts}.csv')

df.to_csv(f, index=False)

print(f"✓ Station Snapshots: {f} ({len(df):,} records)")

print("\n📊 STATION UTILIZATION SUMMARY:")

for station_id in self.CHARGING_STATIONS.keys():

station_data = df[df['station_id'] == station_id]

if len(station_data) > 0:

avg_chrg_util = station_data['charging_utilization_percent'].mean()

avg_wait_util = station_data['waiting_utilization_percent'].mean()

max_chrg = station_data['vehicles_charging'].max()

max_wait = station_data['vehicles_waiting'].max()

print(f"  {station_id}:")

print(f"    Charging: Avg {avg_chrg_util:.1f}% | Peak {int(max_chrg)} vehicles")

print(f"    Waiting:  Avg {avg_wait_util:.1f}% | Peak {int(max_wait)} vehicles")

Station Summary:

- Average utilization %

- Peak occupancy

Other Exports

# Vehicle-Station History

if self.station_vehicle_history:

all_records = []

for station_id, records in self.station_vehicle_history.items():

all_records.extend(records)

if all_records:

df = pd.DataFrame(all_records)

f = os.path.join(self.output_folder, f'vehicle_station_history_{ts}.csv')

df.to_csv(f, index=False)

print(f"✓ Vehicle-Station History: {f} ({len(df):,} records)")

# Vehicle Tracking

if self.vehicle_data:

df = pd.DataFrame(self.vehicle_data)

f = os.path.join(self.output_folder, f'vehicle_tracking_{ts}.csv')

df.to_csv(f, index=False)

print(f"✓ Vehicle Tracking: {f} ({len(df):,} records)")

# Low SOC Alerts

if self.low_soc_alerts:

df = pd.DataFrame(self.low_soc_alerts)

f = os.path.join(self.output_folder, f'low_soc_alerts_{ts}.csv')

df.to_csv(f, index=False)

print(f"✓ Low SOC Alerts: {f} ({len(df):,})")

# Charging Events

if self.charging_events:

df = pd.DataFrame(self.charging_events)

f = os.path.join(self.output_folder, f'charging_events_{ts}.csv')

df.to_csv(f, index=False)

print(f"✓ Charging Events: {f} ({len(df):,})")

if len(df) > 0:

print(f"  Avg Charge Duration: {df['duration_sec'].mean():.1f}s")

print(f"  Avg Wait Time: {df['wait_time_sec'].mean():.1f}s")

print(f"  Avg SOC Gain: {df['soc_gained_percent'].mean():.1f}%")

# Load Demand

if self.load_demand_data:

df = pd.DataFrame(self.load_demand_data)

f = os.path.join(self.output_folder, f'load_demand_{ts}.csv')

df.to_csv(f, index=False)

print(f"✓ Load Demand: {f} ({len(df):,})")

if len(df) > 0:

peak = df['total_power_demand_kW'].max()

avg = df[df['total_power_demand_kW'] > 0]['total_power_demand_kW'].mean()

energy = df['total_power_demand_kW'].sum() / 3600

maxv = df['total_vehicles_charging'].max()

print(f"\n💡 SYSTEM LOAD SUMMARY:")

print(f"  Peak Power: {peak:.2f} kW")

print(f"  Avg Power: {avg:.2f} kW")

print(f"  Total Energy: {energy:.3f} kWh")

print(f"  Max Simultaneous Charging: {int(maxv)} vehicles")

Export All Data: Each dataset to separate CSV file with statistics

21. MAIN FUNCTION

def main():

SUMOCFG = 'Test1.sumocfg'

OUTPUT = 'simulation_outputs'

GUI = True

MAX_TIME = 600

Configuration:

- SUMO config file

- Output folder

- Use GUI

- Run for 600 seconds (10 minutes)

if not os.path.exists(SUMOCFG):

print(f"✗ ERROR: {SUMOCFG} not found!")

return

File Check: Verify config exists

sim = SmartChargingStationMonitor(sumocfg=SUMOCFG, output_folder=OUTPUT)

Create Object: Initialize simulation

if sim.run_simulation(gui=GUI, max_time=MAX_TIME):

sim.export_results()

print("✓ COMPLETE!")

else:

print("✗ FAILED!")

Run: Execute simulation, export results

if __name__ == "__main__":

main()

Entry Point: Run main() when script executed directly

SUMMARY

This code simulates electric vehicle charging station management with:

- 10 charging stations (5 forward, 5 reverse directions)

- Separate charging and waiting areas at each station

- Intelligent routing - finds nearest available station

- Queue management - promotes waiting vehicles when slots open

- Comprehensive data collection - tracks every vehicle, station, charging event

- CSV exports - all data saved for analysis

Key Concepts:

- SOC (State of Charge) - battery percentage

- FIFO queue - first vehicle to wait gets promoted first

- TraCI - Python API to control SUMO simulation

- Timestep - simulation advances in 0.1 second increments