📁 Description of Features in the Generated Output CSV File

The output CSV file stores time-dependent, station-level features collected from the EV charging simulation in Khulna city. Each row represents the state of one charging station at a specific simulation time step.

1️⃣ timestamp

🔹 What

Simulation time (in seconds) at which the data snapshot is recorded.

🔹 Why

EV charging load is time-varying. Timestamp is essential for:

- Time-series modeling

- Load forecasting

- Peak vs off-peak analysis

🔹 How calculated

Directly obtained from the SUMO simulation clock:

timestamp = current simulation step (seconds)

2️⃣ station_id

🔹 What

Unique identifier of the EV charging station.

🔹 Why

- Differentiates multiple charging stations

- Enables station-wise load analysis

- Required for spatial and node-based forecasting

🔹 How calculated

Assigned during charging station creation, e.g.:

CS_0, CS_1, CS_2, ...

This value remains constant for a station across time.

3️⃣ station_edge

🔹 What

The road network edge ID where the charging station is physically located.

🔹 Why

- Links charging demand to traffic flow

- Allows spatial analysis on the road network

- Important for grid–transport interaction modeling

🔹 How calculated

Taken from the SUMO network (.net.xml) during station definition.

Negative edges (e.g., -E0) are sanitized as:

-E0 → NEG_E0

to prevent Excel/formula parsing errors.

4️⃣ connected_vehicles

🔹 What

Number of EVs currently connected and charging at the station.

🔹 Why

- Direct indicator of real-time charging demand

- Strongly correlated with power load

- Helps estimate congestion at charging stations

🔹 How calculated

connected_vehicles = count of EVs actively charging at the station

Retrieved via SUMO’s charging station API.

5️⃣ waiting_vehicles

🔹 What

Number of EVs waiting in queue to access the charging station.

🔹 Why

- Represents unmet charging demand

- Indicates infrastructure inadequacy

- Useful for expansion and optimization studies

🔹 How calculated

waiting_vehicles = total EVs at station − connected EVs

6️⃣ station_capacity

🔹 What

Maximum number of EVs that can charge simultaneously at the station.

🔹 Why

- Physical constraint of charging infrastructure

- Required to model saturation and queuing effects

🔹 How calculated

Defined statically during simulation setup:

station_capacity = number of charging slots

7️⃣ total_energy_consumed (kWh)

🔹 What

Total electrical energy consumed by all EVs at the station up to the current timestamp.

🔹 Why

- Key indicator of energy demand

- Required for grid-level load analysis

- Used to derive consumption trends

🔹 How calculated

total_energy = Σ (energy charged by each EV)

Obtained directly from SUMO’s EV energy tracking.

8️⃣ current_load (kW)

🔹 What

Instantaneous electrical load imposed by the charging station.

🔹 Why

- Primary target variable for load forecasting

- Directly impacts power distribution planning

🔹 How calculated

current_load = number_of_charging_EVs × charger_power_rating

(e.g., 3 kW per EV × connected vehicles)

9️⃣ average_soc

🔹 What

Average State of Charge (SoC) of EVs currently connected to the station.

🔹 Why

- Reflects charging behavior

- Low SoC → longer charging duration → higher load persistence

🔹 How calculated

average_soc = (Σ SoC of connected EVs) / (number of connected EVs)

SoC values are provided by SUMO’s EV battery model.

🔟 energy_demand_remaining (kWh)

🔹 What

Total remaining energy required for all connected EVs to reach full charge.

🔹 Why

- Indicates future short-term load

- Useful for predictive modeling beyond current load

🔹 How calculated

energy_demand_remaining = Σ (battery_capacity − current_energy)

1️⃣1️⃣ time_of_day

🔹 What

Hour of the day derived from simulation time (0–23).

🔹 Why

- Captures daily charging patterns

- Essential for diurnal load forecasting

🔹 How calculated

time_of_day = (timestamp / 3600) % 24

1️⃣2️⃣ is_peak_hour

🔹 What

Binary indicator of peak charging period.

🔹 Why

- Helps models distinguish peak vs off-peak behavior

- Important for demand response analysis

🔹 How calculated

is_peak_hour = 1 if time_of_day ∈ [peak range], else 0

(e.g., 18:00–22:00)

1️⃣3️⃣ traffic_density_near_station

🔹 What

Traffic density on the road segment where the station is located.

🔹 Why

- EV arrivals are traffic-dependent

- Integrates transportation dynamics with power demand

🔹 How calculated

traffic_density = number of vehicles on station_edge / edge_length

1️⃣4️⃣ charging_duration_avg

🔹 What

Average charging time of EVs currently connected at the station.

🔹 Why

- Long charging times increase sustained load

- Useful for queue and congestion modeling

🔹 How calculated

charging_duration_avg = mean(current_time − charging_start_time)

🎯 Summary (for thesis use)

The generated CSV dataset represents a time-series, station-level EV charging load profile, combining traffic, energy, and infrastructure features. These features jointly capture the spatio-temporal dynamics required for accurate EV charging load demand forecasting in Khulna city.

// =============================

Great — these three features are very important, and you’re right to ask this. I’ll explain them cleanly, mathematically, and thesis-ready, and also explicitly link each one to EV load demand forecasting, which examiners love.

I’ll keep the explanation generic to your CSV, so you can paste this directly into your Feature Engineering / Dataset Description section.

Explanation of Derived Features in the Output CSV File

1️⃣ load_per_vehicle_kW

🔹 What it is

load_per_vehicle_kW represents the average instantaneous charging power demand per actively charging electric vehicle at a charging station.

In simple terms, it answers:

“On average, how much power is each connected EV drawing right now?”

🔹 Why this feature is included

- Normalizes total load with respect to the number of charging EVs

- Removes bias caused by station size or EV count

- Helps distinguish behavioral load changes from volume-driven load changes

This is especially important when comparing:

- Small vs large charging stations

- Off-peak vs peak periods

🔹 How it is calculated

[
\text{load_per_vehicle_kW} =
\begin{cases}
\dfrac{\text{current_load (kW)}}{\text{connected_vehicles}}, & \text{if connected_vehicles} > 0 \
0, & \text{otherwise}
\end{cases}
]

Where:

- current_load (kW) = total instantaneous station load

- connected_vehicles = EVs currently charging

🔹 Correlation with load demand prediction

- Directly proportional to instantaneous load

- Indicates charger type (slow vs fast charging)

- Helps models learn power draw behavior independent of EV count

📌 In forecasting models, this feature improves generalization when EV counts fluctuate.

2️⃣ congestion_ratio

🔹 What it is

congestion_ratio measures the level of congestion at a charging station by comparing the number of EVs present to the station’s charging capacity.

It answers:

“How congested is the charging station right now?”

🔹 Why this feature is included

- Captures charging infrastructure stress

- Reflects unmet or delayed charging demand

- Helps predict future load surges caused by queued EVs

🔹 How it is calculated

[
\text{congestion_ratio} =
\dfrac{\text{connected_vehicles} + \text{waiting_vehicles}}
{\text{station_capacity}}
]

Where:

- station_capacity = maximum simultaneous chargers

Values interpretation:

- ≈ 1.0 → station fully utilized

- > 1.0 → queue exists (overloaded station)

- < 1.0 → underutilized station

🔹 Correlation with load demand prediction

- High congestion today ⇒ higher future load

- Indicates deferred charging demand

- Helps forecasting models capture lagged demand effects

📌 Particularly useful for short-term load forecasting.

3️⃣ demand_to_capacity_ratio

🔹 What it is

demand_to_capacity_ratio represents the ratio between the total remaining energy demand and the station’s maximum energy delivery capability over a given time horizon.

It answers:

“How much energy is demanded relative to what the station can realistically deliver?”

🔹 Why this feature is included

- Combines energy demand and infrastructure limits

- Reflects sustainability of current load levels

- Indicates whether load pressure will persist over time

🔹 How it is calculated

A common formulation used in your CSV:

[
\text{demand_to_capacity_ratio} =
\dfrac{\text{energy_demand_remaining (kWh)}}
{\text{station_capacity} \times \text{charger_power (kW)}}
]

Where:

- energy_demand_remaining = total kWh still required by connected EVs

- charger_power = rated power per charging slot

🔹 Correlation with load demand prediction

- High ratio ⇒ prolonged high load expected

- Captures persistence of charging demand

- Helps models learn load duration, not just load magnitude

📌 Extremely valuable for medium-term forecasting horizons.

🔗 How These Features Support Load Demand Forecasting (Summary Table)

| Feature | Captures | Contribution to Load Prediction |
| --- | --- | --- |
| load_per_vehicle_kW | Per-EV charging behavior | Improves power scaling accuracy |
| congestion_ratio | Station stress & queuing | Predicts near-future load spikes |
| demand_to_capacity_ratio | Energy pressure vs infrastructure | Models load persistence over time |

🧠 Thesis-Ready Insight (You can quote this)

These derived features collectively transform raw charging and traffic data into behavior-aware, capacity-aware indicators, enabling more accurate and robust EV charging load demand forecasting under varying traffic and infrastructure conditions in Khulna city.