📊 Load Demand Calculation Explanation

Based on output data, here's exactly how the load demand values are calculated:

🔢 Step-by-Step Calculation Process

At Each Timestep (Every Second):

The script checks every vehicle in the simulation and asks:

"Is this vehicle currently charging at a charging station?"

# For each vehicle:

charging_station_id = traci.vehicle.getParameter(veh_id, "device.battery.chargingStationId")

if charging_station_id != "NULL":

# Vehicle IS charging

power_kW = CHARGING_STATIONS[charging_station_id]['power_kW']

total_power_demand += power_kW

📈 Reading Your Data:

Timestep 72-94: First Vehicle Charging

timestep_sec = 72

total_vehicles_charging = 1

total_power_demand_kW = 5.0

CS_Node2_vehicles = 1

CS_Node2_power_kW = 5.0

What happened:

- 1 vehicle started charging at CS_Node2 (5 kW station)

- Calculation: 1 vehicle × 5 kW = 5 kW total load

- Duration: 72s to 94s = 22 seconds of charging

- Energy consumed: 5 kW × 22 sec / 3600 = 0.031 kWh

Timestep 95-132: Second Vehicle Joins

timestep_sec = 95

total_vehicles_charging = 2

total_power_demand_kW = 10.0

CS_Node2_vehicles = 2

CS_Node2_power_kW = 10.0

What happened:

- Vehicle 1 still charging at CS_Node2 (5 kW)

- Vehicle 2 also charging at CS_Node2 (5 kW)

- Calculation: 2 vehicles × 5 kW = 10 kW total load

- Duration: 95s to 132s = 37 seconds at 10 kW

- Energy: 10 kW × 37 sec / 3600 = 0.103 kWh

Timestep 133-147: Third Vehicle at Different Station

timestep_sec = 133

total_vehicles_charging = 3

total_power_demand_kW = 17.0

CS_Node2_vehicles = 2

CS_Node2_power_kW = 10.0

CS_Node3_E1_vehicles = 1

CS_Node3_E1_power_kW = 7.0

What happened:

- Vehicles 1 & 2 still at CS_Node2: 2 × 5 kW = 10 kW

- Vehicle 3 started at CS_Node3_E1: 1 × 7 kW = 7 kW

- Total calculation: 10 kW + 7 kW = 17 kW

🧮 Mathematical Formula:

Total Load at Timestep t:

Load_Demand(t) = Σ [Vehicle_i charging at time t] × [Station_Power]

For timestep 133:

= (CS_Node2: 2 vehicles × 5 kW) + (CS_Node3_E1: 1 vehicle × 7 kW)

= 10 kW + 7 kW

= 17 kW

Per-Station Load:

Station_Load(t) = Number_of_Vehicles_Charging × Station_Power_Rating

CS_Node2 at t=133:

= 2 vehicles × 5 kW/vehicle

= 10 kW

CS_Node3_E1 at t=133:

= 1 vehicle × 7 kW/vehicle

= 7 kW

💡 Key Points:

1. Power is Constant During Charging

- When a vehicle charges at 5 kW station → it draws exactly 5 kW continuously

- When a vehicle charges at 7 kW station → it draws exactly 7 kW continuously

- No variation during charging session

2. Load is Additive

If 3 vehicles charge at same 5 kW station:

Total station load = 3 × 5 = 15 kW

If 2 vehicles at 5 kW + 1 vehicle at 7 kW:

Total system load = (2 × 5) + (1 × 7) = 17 kW

3. Zero Load When No Charging

Timesteps 1-71: total_power_demand_kW = 0

Reason: No vehicles were charging yet

(They're driving or haven't reached low SOC threshold)

📊 Energy Calculation:

From Your Data (Timesteps 72-147):

Session 1 (CS_Node2, 1 vehicle, 72-94s):

Energy = Power × Time

= 5 kW × 22 seconds / 3600

= 0.031 kWh

Session 2 (CS_Node2, 2 vehicles, 95-132s):

Energy = 10 kW × 37 seconds / 3600

= 0.103 kWh

Session 3 (Mixed, 3 vehicles, 133-147s):

Energy = 17 kW × 14 seconds / 3600

= 0.066 kWh

Total Energy (so far):

0.031 + 0.103 + 0.066 = 0.200 kWh

🎯 Summary:

| Timestep | Vehicles Charging | Where | Power Calculation | Total Load |
| --- | --- | --- | --- | --- |
| 1-71 | 0 | - | 0 | 0 kW |
| 72-94 | 1 | CS_Node2 | 1 × 5 | 5 kW |
| 95-132 | 2 | CS_Node2 | 2 × 5 | 10 kW |
| 133-147 | 3 | CS_Node2 + CS_Node3_E1 | (2×5) + (1×7) | 17 kW |

🔍 Why These Specific Values?

5 kW and 7 kW Power Ratings:

From your charging station configuration:

<!-- CS_Node2 -->

<chargingStation power="5000" />  <!-- 5000 W = 5 kW -->

<!-- CS_Node3_E1 -->

<chargingStation power="7000" />  <!-- 7000 W = 7 kW -->

Charging Start Times (72s, 95s, 133s):

- Vehicles start with ~32% SOC

- Drive until SOC drops below 30% threshold

- Script detects low SOC → routes to nearest station

- Vehicle arrives and starts charging

- Each vehicle takes different time based on:

  - Initial SOC

  - Route taken

  - Speed

  - Traffic conditions

📌 In Simple Terms:

Load Demand = How many vehicles are plugged in × Power per charger

It's like counting how many phones are charging:

- 1 phone at 5W charger = 5W load

- 2 phones at 5W chargers = 10W load

- Add 1 more at 7W charger = 17W total load

Your simulation counts EV charging the same way! 🔌⚡