📘 Load Demand in EV Charging Systems - Theoretical Knowledge

🎯 What is Load Demand?

Load demand in the context of EV charging infrastructure refers to the total electrical power required by all vehicles charging simultaneously at charging stations within a specific time period.

🔋 Core Concepts

1. Individual Vehicle Charging Load

For a single vehicle:

Charging Load (kW) = Charging Power × Number of Vehicles

Example:

- 1 E-Rickshaw charging at 5 kW station = 5 kW load

- 3 E-Vans charging at 7 kW stations = 3 × 7 = 21 kW load

2. Aggregate Load Demand

The total load at any given time:

Total Load Demand (t) = Σ [Charging Power of Station_i × Occupancy_i(t)]

Where:

- Charging Power: Power rating of each station (5 kW, 7 kW, etc.)

- Occupancy: Whether station is in use (1) or idle (0)

- t: Time instant

📊 Types of Load Demand

A. Instantaneous Load Demand

Power required at a specific moment in time.

P_instant(t) = Σ P_station_i(t)  [at time t]

Example at t=150s:

- CS_Node2: 1 vehicle × 5 kW = 5 kW

- CS_Node3_E1: 2 vehicles × 7 kW = 14 kW

- Total: 19 kW

B. Peak Load Demand

Maximum power required during the entire simulation period.

P_peak = MAX(P_instant(t))  [for all t]

Critical for:

- Transformer sizing

- Grid capacity planning

- Circuit breaker ratings

C. Average Load Demand

Mean power consumption over the study period.

P_avg = (Σ P_instant(t)) / Total_Time

Critical for:

- Energy billing

- Grid planning

- Battery storage sizing

D. Energy Demand

Total energy consumed over time period.

Energy_total (kWh) = ∫ P(t) dt = Σ [P_instant(t) × Δt]

🏗️ Components of Load Demand Calculation

1. Station-Level Parameters

| Parameter | Symbol | Unit | Description |
| --- | --- | --- | --- |
| Rated Power | P_rated | kW | Maximum output of station |
| Efficiency | η | % | Energy transfer efficiency |
| Actual Power | P_actual | kW | P_rated × η × Occupancy |

2. Time-Varying Factors

a) Occupancy Rate

Occupancy(t) = Number of vehicles charging / Total stations

b) Utilization Factor

Utilization = Total charging time / Total available time

c) Load Factor

Load Factor = Average Load / Peak Load

- High load factor (>0.7) = Efficient use

- Low load factor (<0.5) = Underutilized/peaky demand

📐 Mathematical Formulations

1. Instantaneous Load at Station i

P_station_i(t) = {

P_rated_i × η_i  if vehicle is charging

0                if station is idle

}

2. Network-Wide Instantaneous Load

P_total(t) = Σ(i=1 to N) P_station_i(t)

Where N = total number of stations

3. Peak Demand

P_peak = MAX{P_total(t) | t ∈ [0, T_sim]}

4. Average Demand

P_avg = (1/T_sim) × ∫(0 to T_sim) P_total(t) dt

For discrete data:

P_avg = (Σ P_total(t_k) × Δt) / T_sim

5. Total Energy

E_total = ∫(0 to T_sim) P_total(t) dt = Σ P_total(t_k) × Δt

🌐 System-Level Load Demand

Coincidence Factor

Not all stations charge at full power simultaneously:

P_system = Coincidence Factor × Σ P_rated_i

Typical Values:

- High traffic: CF = 0.6 - 0.8

- Medium traffic: CF = 0.4 - 0.6

- Low traffic: CF = 0.2 - 0.4

🔍 Load Demand Metrics (What You Need to Calculate)

From Your Simulation Data:

| Metric | Formula | Purpose |
| --- | --- | --- |
| Instantaneous Load | P(t) at each second | Load profile graph |
| Peak Load | MAX(P(t)) | Transformer sizing |
| Average Load | MEAN(P(t)) | Energy planning |
| Total Energy | SUM(P(t) × Δt) | Cost estimation |
| Station Utilization | Charging time / Sim time | Efficiency analysis |
| Load Duration Curve | Sorted P(t) values | Capacity planning |
| Simultaneity | Peak load / (N × P_rated) | Coincidence factor |

📈 Load Demand Profile

A load profile shows how demand varies over time:

^

Power |     Peak

(kW)  |      ╱╲

|     ╱  ╲___

|____╱       ╲____

+-------------------> Time

0    t_peak   T

Key Features:

- Ramp-up period: Increasing demand as vehicles arrive

- Peak period: Maximum simultaneous charging

- Steady state: Stable demand

- Ramp-down: Declining demand as vehicles leave

🎓 Example Scenario

Given:

- 3 stations: 5 kW, 7 kW, 7 kW

- Time period: 0-600s

- Charging events captured every second

At t = 100s:

- Station 1 (5 kW): Charging (1 vehicle)

- Station 2 (7 kW): Idle

- Station 3 (7 kW): Charging (1 vehicle)

Calculations:

P_instant(100) = (5 kW × 1) + (7 kW × 0) + (7 kW × 1) = 12 kW

If this repeats for 120 seconds:

E_consumed = 12 kW × (120/3600) h = 0.4 kWh

🏭 Practical Applications

1. Grid Planning

- Peak demand → Size transformers

- Average demand → Estimate grid impact

- Load factor → Assess efficiency

2. Cost Analysis

- Energy demand (kWh) → Calculate electricity cost

- Peak demand (kW) → Demand charges (if applicable)

3. Infrastructure Design

- Utilization rate → Decide if more stations needed

- Load curve → Optimize station placement

4. Battery Storage Sizing

Battery Capacity = Peak Load × Discharge Duration

= 50 kW × 2 hours = 100 kWh

📊 Your Simulation Context

What You Have:

- 7 Charging Stations with known power ratings

- Time-series data of vehicles at each station

- Charging durations and SOC changes

- Simulation period: 600 seconds

What You Can Calculate:

Station-Level:

- Individual station load over time

- Station utilization (%)

- Energy delivered per station

Network-Level:

- Total instantaneous load P(t)

- Peak load demand (kW)

- Average load demand (kW)

- Total energy consumed (kWh)

- Load factor

- Load duration curve

Vehicle-Level:

- Energy demand per vehicle type

- Charging frequency

- Average charging duration

🧮 Data Requirements for Calculation

From your simulation outputs, you need:

| Data Item | Source | Use |
| --- | --- | --- |
| Charging station ID | chargingStationId | Identify active stations |
| Station power rating | XML config | Calculate P_station |
| Timestamp | timestep_sec | Time-series analysis |
| Charging status | is_charging flag | Determine occupancy |
| Vehicle type | vehicle_type | Categorize demand |
| SOC before/after | arrival_soc, departure_soc | Validate energy |

🎯 Summary

Load Demand = Power × Time × Occupancy

Key Equations:

- Instantaneous: P(t) = Σ [P_station × Occupancy(t)]

- Peak: P_peak = MAX(P(t))

- Average: P_avg = MEAN(P(t))

- Energy: E = Σ [P(t) × Δt]

Purpose:

- Grid impact assessment

- Infrastructure planning

- Cost estimation

- System optimization

Would you like me to now show you how to implement these calculations using your simulation data?

I can create:

- ✅ Load demand calculator script

- ✅ Visualization of load profiles

- ✅ Station utilization analysis

- ✅ Peak/average/energy metrics

Let me know when you're ready! 🚀