Network Structure (Test1.net.xml)

Nodes (Junctions):

- 13 nodes total (Node1 through Node13)

- Node2 and Node6 have traffic lights

- Network spans from coordinates (-931.99, -386.56) to (1284.39, 595.45)

Edges (Roads):

- Multiple routes connecting nodes with various lengths:

  - E0: Node1→Node2 (850m)

  - E1: Node2→Node3 (800m)

  - E2: Node3→Node4 (94m)

  - E3: Node4→Node13 (1300m)

  - And many other connecting edges

Key Characteristics:

- Single lane per edge (index="0")

- Speed limit: 13.89 m/s (~50 km/h) for most edges

- Some priority junctions, some traffic-light controlled

Electric Vehicle Charging Network (Test1.add.xml)

Charging Station Types:

- Fast Chargers (50-75 kW): At N2, N8

- Ultra-Fast (150 kW): At N3

- Standard (22 kW): At N5

- Medium (30 kW): At N6

- E-Bus Depot (200 kW): At N3

- E-Rickshaw Hubs (5 kW): At N2, N5

- E-Bike Battery Swap (3 kW): At N1, N6

Additional Infrastructure:

- EV parking areas at N2, N3, N5

- Bus stops for electric buses at N2, N3, N6

- Range extenders at 860m and 900m marks

Vehicle Routes (Test1.rou.xml)

Vehicle Types:

- Easy Bikes (e-bikes): Max speed 19.44 m/s, length 2.2m

- E-Rickshaws: Max speed 6.94 m/s, length 2.5m

- E-Vans: Max speed 13.89 m/s, length 2.5m

8 Different Routes from N1 to N13:

- Route 1 (Direct): E0→E1→E2→E3 (shortest)

- Routes 2-8: Various alternatives via N5, N6, N7, N8, N9, N10, N11

Traffic Distribution:

- 60% Easy Bikes (flows 35-80 veh/hr per route)

- 25% E-Rickshaws (flows 22-35 veh/hr)

- 15% E-Vans (flows 15-22 veh/hr)

Observations & Potential Issues:

- Bangladesh Context: Good localization with e-rickshaws and appropriate vehicle types

- Charging Coverage: Well-distributed across the network

- Route Diversity: 8 different paths provide good alternatives

- Traffic Volume: Moderate flows starting at different times to avoid initial congestion

1. Network File (Test1.net.xml) Units

Location/Coordinates

- Unit: meters (m)

- Examples:

  - convBoundary="-931.99,-386.56,1284.39,595.45" → bounding box in meters

  - Junction positions: x="-931.99" y="595.45" → coordinates in meters

Edge/Lane Properties

- length: meters (m)

  - Example: length="850.00" = 850 meters

- speed: meters per second (m/s)

  - Example: speed="13.89" = 13.89 m/s = 50 km/h

  - Conversion: multiply by 3.6 to get km/h

- index: dimensionless integer

  - Lane numbering (0, 1, 2, etc.)

Vehicle Type Properties

- accel: meters per second² (m/s²)

  - Example: accel="1.2" = 1.2 m/s² acceleration

- decel: meters per second² (m/s²)

  - Example: decel="4.5" = 4.5 m/s² deceleration (braking)

- sigma: dimensionless (0-1)

  - Driver imperfection parameter (0=perfect, 1=very imperfect)

- length: meters (m)

  - Vehicle length: length="2.2" = 2.2 meters

- width: meters (m)

  - Vehicle width: width="0.8" = 0.8 meters

- height: meters (m)

  - Vehicle height: height="1.4" = 1.4 meters

- maxSpeed: meters per second (m/s)

  - Example: maxSpeed="19.44" = 19.44 m/s = 70 km/h

- minGap: meters (m)

  - Minimum gap to leading vehicle: minGap="0.5" = 0.5 meters

- tau: seconds (s)

  - Desired time headway: tau="0.9" = 0.9 seconds

- speedFactor: dimensionless multiplier

  - speedFactor="1.05" = drives at 105% of speed limit

Traffic Light Timing

- duration: seconds (s)

  - Example: duration="42" = 42 seconds green phase

2. Charging Station File (Test1.add.xml) Units

Position on Lane

- startPos: meters (m) from lane start

  - Example: startPos="400" = 400 meters from beginning of lane

- endPos: meters (m) from lane start

  - Example: endPos="420" = 420 meters from beginning

Electrical Properties

- power: Watts (W)

  - Example: power="50000" = 50,000 W = 50 kW

  - Common values:

    - 3,000 W = 3 kW (e-bike)

    - 22,000 W = 22 kW (standard EV)

    - 50,000 W = 50 kW (fast charger)

    - 150,000 W = 150 kW (ultra-fast)

- efficiency: dimensionless (0-1)

  - Example: efficiency="0.92" = 92% efficient

  - Energy delivered = power × efficiency

Parking Areas

- roadsideCapacity: number of spaces (dimensionless)

  - Example: roadsideCapacity="15" = 15 parking spots

3. Route File (Test1.rou.xml) Units

Time Units

- depart: seconds (s) from simulation start

  - Example: depart="0" = starts at time 0

  - depart="10" = starts at 10 seconds

- begin: seconds (s) - flow start time

  - Example: begin="50" = flow begins at 50 seconds

- end: seconds (s) - flow end time

  - Example: end="200" = flow ends at 200 seconds

Flow Rate

- vehsPerHour: vehicles per hour (veh/h)

  - Example: vehsPerHour="80" = 80 vehicles/hour

  - Conversion: 80 veh/h = 1.33 veh/minute = 1 vehicle every 45 seconds

4. Common SUMO Output Files Units

Tripinfo Output (--tripinfo-output)

<tripinfo id="vehicle_01"

depart="5.00"              <!-- seconds -->

departLane="E0_0"          <!-- lane ID -->

departPos="5.10"           <!-- meters -->

departSpeed="0.00"         <!-- m/s -->

departDelay="0.00"         <!-- seconds -->

arrival="125.00"           <!-- seconds -->

arrivalLane="E3_0"         <!-- lane ID -->

arrivalPos="1294.90"       <!-- meters -->

arrivalSpeed="13.89"       <!-- m/s -->

duration="120.00"          <!-- seconds -->

routeLength="3044.00"      <!-- meters -->

waitingTime="12.00"        <!-- seconds -->

waitingCount="3"           <!-- number of stops -->

stopTime="0.00"            <!-- seconds -->

timeLoss="18.50"           <!-- seconds -->

rerouteNo="0"              <!-- number of reroutes -->

devices="tripinfo"         <!-- device list -->

vType="easyBike"           <!-- vehicle type -->

speedFactor="1.05"         <!-- dimensionless -->

vaporized=""/>             <!-- boolean -->

Key Metrics:

- duration: Total travel time (seconds)

- routeLength: Total distance traveled (meters)

- timeLoss: Time lost vs. ideal travel time (seconds)

- waitingTime: Time spent with speed < 0.1 m/s (seconds)

Emissions Output (--emission-output)

<timestep time="10.00">              <!-- seconds -->

<vehicle id="vehicle_01"

eclass="Energy/unknown"

CO2="0.00"                <!-- mg/s (milligrams/second) -->

CO="0.00"                 <!-- mg/s -->

HC="0.00"                 <!-- mg/s -->

NOx="0.00"                <!-- mg/s -->

PMx="0.00"                <!-- mg/s -->

fuel="0.00"               <!-- ml/s (milliliters/second) -->

electricity="2500.00"     <!-- Wh/s = W (Watts) -->

noise="60.5"              <!-- dB (decibels) -->

route="route1"

type="easyBike"

waiting="0.00"            <!-- seconds -->

lane="E0_0"

pos="150.20"              <!-- meters -->

speed="12.50"             <!-- m/s -->

angle="90.00"             <!-- degrees (0-360) -->

x="-800.50"               <!-- meters -->

y="450.30"/>              <!-- meters -->

</timestep>

Energy Units:

- electricity: Watts (W) - instantaneous power consumption

- For EVs: typically 0-30,000 W depending on speed/acceleration

Battery Output (--battery-output)

<timestep time="10.00">              <!-- seconds -->

<vehicle id="easybike_01"

actualBatteryCapacity="15000"  <!-- Wh (Watt-hours) -->

maximumBatteryCapacity="20000" <!-- Wh -->

chargingStationId="fast_charger_n2"

energyConsumed="50.5"          <!-- Wh -->

energyCharged="0.0"            <!-- Wh -->

speed="12.5"                   <!-- m/s -->

acceleration="0.5"             <!-- m/s² -->

x="-800.5"                     <!-- meters -->

y="450.3"                      <!-- meters -->

lane="E0_0"

posOnLane="150.2"              <!-- meters -->

timeStopped="0"/>              <!-- seconds -->

</timestep>

Battery Metrics:

- Wh (Watt-hours): Energy capacity/consumption

- Example: 15,000 Wh = 15 kWh battery

FCD Output (Floating Car Data)

<timestep time="10.00">              <!-- seconds -->

<vehicle id="easybike_01"

x="-800.50"               <!-- meters -->

y="450.30"                <!-- meters -->

angle="90.00"             <!-- degrees -->

type="easyBike"

speed="12.50"             <!-- m/s -->

pos="150.20"              <!-- meters on edge -->

lane="E0_0"

slope="0.00"/>            <!-- degrees (incline) -->

</timestep>

Summary Output (--summary-output)

<step time="100.00"                  <!-- seconds -->

loaded="50"                    <!-- vehicles loaded -->

inserted="48"                  <!-- vehicles inserted -->

running="35"                   <!-- vehicles running -->

waiting="2"                    <!-- vehicles waiting -->

ended="13"                     <!-- vehicles ended trip -->

meanWaitingTime="5.20"         <!-- seconds -->

meanTravelTime="120.50"        <!-- seconds -->

meanSpeed="10.25"              <!-- m/s -->

meanSpeedRelative="0.74"       <!-- fraction of max speed -->

duration="100000"              <!-- milliseconds (wall time) -->

halting="8"                    <!-- vehicles with v < 0.1 m/s -->

meanHaltingDuration="3.50"/>   <!-- seconds -->

5. Unit Conversions for Reference

Speed

- 1 m/s = 3.6 km/h

- 13.89 m/s = 50 km/h

- 27.78 m/s = 100 km/h

Energy

- 1 kWh = 1000 Wh

- 1 W = 1 J/s (Joule per second)

- Power (W) × Time (h) = Energy (Wh)

Distance

- 1 km = 1000 m

- Lane position is cumulative along edge

Time

- SUMO uses seconds as base unit

- Output timestep typically 1.0 second (configurable)

6. Your Specific File Values

From your files:

- E-Bike: 19.44 m/s max = 70 km/h

- E-Rickshaw: 6.94 m/s max = 25 km/h

- E-Van: 13.89 m/s max = 50 km/h

- Ultra-fast charger: 150,000 W = 150 kW

- Total network distance (Route 1): 850+800+94+1300 = 3,044 meters = 3.04 km