Rising demand of battery-powered vehicles for sustainable environment of Bangladesh: A review of challenges and opportunities in Bangladesh

6. Challenges and Barriers (Rakesh Biswas)

6.1Energy & Infrastructure

Grid Capacity and Power Supply Concerns

Bangladesh's electric vehicle ecosystem faces significant challenges of energy infrastructure. The country's electricity grid is already under pressure from existing demand. Again, it is facing additional pressure due to EV charging requirements. Electric three-wheelers consume about 5% of Bangladesh's total electricity consumption. Most of which comes from fossil fuel sources such as natural gas, potentially neglecting some of the environmental benefits of e-mobility. It means that EVs are only as environmentally friendly as the source of power generation.

The current erratic power supply in Bangladesh is a major hurdle for EV adoption. Frequent load shedding creates serious reliability concerns for owners who depend on a steady flow of electricity to charge their vehicles. These outages don't just inconvenience users; they also erode confidence in electric vehicle technology as a practical and dependable transport solution.

Reference: [1] Low Carbon Power. (2025). Bangladesh Electricity Generation Mix 2024/2025. Available at: https://lowcarbonpower.org/region/Bangladesh

[2] Hossain, M. S., et al. (2023). Affordable Electric Three-Wheeler in Bangladesh: Prospects, Challenges, and Sustainable Solutions. Sustainability, 15(1), 149. Available at: https://www.mdpi.com/2071-1050/15/1/149

[3] Energy Tracker Asia. (2024). Bangladesh's Energy Scenario in 2024. Available at: https://energytracker.asia/bangladesh-energy-scenario/

Charging Infrastructure Deficiency

As of 2024, the country only has 14 government-approved stations with a total capacity of 282 kW. Worse, most are solar-powered, which means you're looking at extremely long charge times—totally unusable for commercial vehicles that need a quick turnaround. These stations are tightly clustered in Dhaka and other big cities. They can be located at places like Audi Bangladesh in Tejgaon and the Sumatra Filling Station in Cantonment, with others scattered in Comilla, Narayanganj, Chittagong, Gazipur, and Jessore. The required charging time highlights additional operational or functional complications. For example, battery-powered rickshaws and easy bikes, require a notably long charging duration of six to eight hours for a full cycle. This results in a daily expenditure estimated between 120 and 150 BDT, with these costs steadily rising annually due to increasing electricity tariffs. While home charging is an alternative solution, it necessitates the installation of specialized high-voltage electrical connections. The consumer is fully responsible for bearing the entirety of these associated installation costs. Moreover, the electricity utilized for this charging is levied at the commercial rate, which is significantly higher than the standard residential tariff.

Reference: (1) https://lightcastlepartners.com/insights/2023/08/dissecting-the-mujib-climate-prosperity-plan/#:~:text=At%20present%2C%20there%20are%20only,as%20well%20as%20parking%20lots.

(2) https://cpd-power-energy-study.com/realigning-the-existing-ev-policies-of-bangladesh-in-line-with-the-industrys-needs/#:~:text=The%20underdeveloped%20infrastructure%20of%20Bangladesh%20acts%20as,currently%20just%2014%20solar%2Dpowered%20EV%20charging%20stations.

(3) BBF Digital. (2024). Bangladesh's Bold Leap Towards an Electric Vehicle Future. Available at: https://bbf.digital/bangladeshs-bold-leap-towards-an-electric-vehicle-future

(4) Dhaka Tribune. (2024). Electric vehicle boom stalls as charging infrastructure faces setbacks in Bangladesh. Available at: https://www.dhakatribune.com/bangladesh/350558/electric-vehicle-boom-stalls-as-charging [9] Ibid.

Renewable Energy Integration Limitations

In 2024, Bangladesh's renewable energy capacity showed strong growth, increasing by 42.7% and adding about 331 MW of power to the main grid. However, this progress is at risk because there are very few investment projects ready for 2025-2026. This could limit growth in the future. The country has a big goal to triple its renewable energy capacity by 2030, which means it needs to add 3,000 megawatts of new projects.
Unfortunately, only about 500 MW of these projects are currently in the advanced stages of construction. This large difference between the country’s goal and the current pace of development severely restricts the growth of a genuinely environmentally friendly EV ecosystem.

Reference: (1) https://ieefa.org/resources/new-year-same-goal-bangladesh-needs-clear-existing-roadblocks-clean-energy#:~:text=In%202024%2C%20Bangladesh's%20renewable%20energy,adding%203%2C000MW%20renewable%20energy.

(2) The Business Standard. (2025). Bangladesh's renewable energy capacity grows by 42.7% in 2024. January 15, 2025. Climate Adaptation Platform. (2025). Bangladesh Bolsters Energy Future with Solar Power Investments. Available at: https://climateadaptationplatform.com/bangladesh-bolsters-energy-future-with-solar-power-investments/

(3) IEEFA. (2023). Charting an electricity sector transition pathway for Bangladesh. Available at: https://ieefa.org/resources/charting-electricity-sector-transition-pathway-bangladesh.

6.2Regulatory

Absence of Formal Recognition and Standardization

One of the main regulatory issues is that electric three-wheelers, the most common type of EV in Bangladesh with 3-4 million vehicles, which aren't officially recognized. The National Electric Vehicle policy and the regulations of the Bangladesh Road Transport Authority (BRTA) do not cover them. Due to this- the sector operates informally without proper oversight or quality control. Research shows common issues like bad brakes, suspension problems, and structural weaknesses that can cause rollovers. Since there are no required safety certifications, these unsafe vehicles continue to operate on public roads, putting drivers and others at risk.

Reference:

- https://smepprogramme.org/the-electric-three-wheeler-sector-in-bangladesh-regulating-for-growth-preventing-lead-pollution-and-unlocking-green-investment/#:~:text=Electric%20three%2Dwheelers%20%E2%80%93%20EZ%20bikes,lead%20recycling%20practices%20are%20widespread.

- https://restofworld.org/2024/bangladesh-e-rickshaw-legal-dhaka/

- https://en.prothomalo.com/bangladesh/4v53ok95ap#:~:text=According%20to%20BRTA%2C%20passenger%20rights,on%20operation%20of%20these%20vehicles.

Policy Fragmentation and Coordination Gaps

The clean energy sector is facing issues due to a lack of coordination between government organizations and private companies, and many people involved don't have the necessary skills. Many regulatory bodies- such as SREDA, BRTA, Bangladesh Energy Regulatory Commission, and the Ministry of Power, Energy and Mineral Resources, are in charge of different parts of the electric vehicle industry. However, these bodies don't coordinate well. This lack of coordination leads to bureaucratic inefficiencies and conflicting requirements, which hinders the industry's progress.

Reference:

- https://file-khulna.portal.gov.bd/uploads/d07c3114-12a6-4ebe-8fa0-460e27018e82//671/5a0/75b/6715a075bfbf0518800500.pdf

- https://idcol.org/download/EV_Financing_Toolkit.pdf

- https://sreda.gov.bd/

6.3Battery & Waste Management

Lead-Acid Battery Dominance and Associated Hazards

Each easy bike battery set is incredibly heavy, holding about 125 kg of lead. That is 15 times more lead than what is found in a standard car battery. Since these lead-acid batteries only last 8 to 11 months, they generate a staggering amount of waste: over 167,000 metric tons of lead every year in Bangladesh. This rapid need to replace batteries creates a huge, continuous waste management problem. Battery manufacturing and recycling are handled by both registered ("formal") and unregistered ("informal") businesses. The informal groups often recover lead using simple, coal-based open-pit furnaces that have no equipment to control pollution. While formal recycling facilities are much better, losing only 1,134 tons of lead, informal recycling is responsible for an enormous loss of 6,924 tons of lead in 2021 alone. This means 86% of all lead loss comes from informal recycling.

Reference:

- https://unctad.org/meeting/workshop-business-innovation-and-policy-solutions-informal-recycling-used-lead-acid#:~:text=One%20EZ%20Bike%20alone%20contains%20125%20kg,over%20167%2C000%20MT%20of%20lead%20waste%20annually.

(2) Pure Earth & UNICEF. (2022). The Toxic Truth: Children's Exposure to Lead Pollution Undermines a Generation of Future Potential. Available at: https://www.unicef.org/reports/toxic-truth-childrens-exposure-to-lead-pollution-2020

(3) https://unctad.org/system/files/non-official-document/%5B_SMEP_%5D_ULABs_26-03-2024.pdf

(4) https://www.tbsnews.net/features/panorama/vicious-toxic-lead-cycle-illegal-lead-battery-factories-pollution-and-unauthorised

(5) https://smepprogramme.org/project/solutions-to-informal-recycling-of-used-lead-acid-batteries/#:~:text=Background%20Information,affected%20by%20lead%20pollution%20worldwide.

(6) https://www.sciencedirect.com/science/article/abs/pii/S2352550924001593#:~:text=In%20dissecting%20the%20flow%20of,Pb%2C%20revealing%20how%20the%20LAB

(7) https://batterycouncil.org/news/press-release/new-study-confirms-lead-batteries-maintain-remarkable-99-recycling-rate/#:~:text=%E2%80%9CTogether%2C%20we'll%20continue,circular%20economy%20for%20lead%20batteries.

(8) https://www.nvclppp.org/bulletin/lead-poisoning-crisis-in-bangladesh-tied-to-informal-battery-recycling/

Environmental and Health Impacts

Since most lead waste is recycled in those informal open-pit furnaces, it causes serious lead pollution. This contamination poisons the workers directly and the people living nearby. Shockingly, two out of every three children in Bangladesh now have dangerously high levels of lead in their blood. This kind of poisoning leads to problems like IQ loss, worse educational outcomes, lower future earnings, and severe heart and kidney diseases. Studies show that over 20% of the population lives within 5 km of an informal smelting site, and people living there see a 6-percentage point rise in terminated pregnancies.

Beyond the direct poisoning, the informal making of EV batteries is a major cause of problems like land use damage, global warming, air pollution (fine particulate matter), and acid rain. Global warming and air pollution each make up about 40% of the total health risks caused by this industry. Workers in the informal sector face an alarming risk of developing toxic diseases. The majority of these non-cancer risks—90.8%—come from lead entering the body through the mouth.

Reference:

- https://www.unicef.org/bangladesh/en/press-releases/alarming-rate-blood-lead-levels-among-children-unicef-urges-interim-government#:~:text=Press%20release-,An%20alarming%20rate%20of%20blood%20lead%20levels%20among%20children:%20UNICEF,UNICEF%20Bangladesh/2024/Salauddin

- https://www.unicef.org/bangladesh/en/press-releases/third-worlds-children-poisoned-lead-bangladesh-fourth-most-seriously-hit-terms#:~:text=The%20report%2C%20the%20first%20of,Hozumi%2C%20UNICEF%20Representative%20in%20Bangladesh.

- https://www.unicef.org/rosa/stories/lead-poisoning#:~:text=UNICEF/UN0580740/Pelden%20Bhutan's%20National,threatening%20levels%20of%20lead%20poisoning.

- https://www.who.int/news-room/fact-sheets/detail/lead-poisoning-and-health#:~:text=Lead%20exposure%20can%20have%20serious,toxicity%20to%20the%20reproductive%20organs.

- https://nchh.org/policy/lead-exposure-and-education-results/#:~:text=Lead%20poisoning%20has%20serious%20implications,grade%20tests%20in%20elementary%20school.

- https://www.researchgate.net/publication/381037965_Patching_sustainability_loopholes_within_the_lead-acid_battery_industry_of_Bangladesh_An_environmental_and_occupational_health_risk_perspective

- https://www.pureearth.org/lead-poisoning-and-car-batteries-initiative/#:~:text=The%20unawareness%20of%20the%20dangers,airborne%20and%20travel%20long%20distances.

Regulatory Vacuum in Battery Management

There is a major gap in the rules. While the national hazardous e-waste policy covers household batteries, it completely ignores used lead-acid batteries from electric three-wheelers. Because the vehicle and battery sectors are mostly informal and unregulated, the government loses huge amounts of money in taxes. Only 30% of lead-acid battery recycling is formal, which results in tax revenue losses of about US$91 million every year. Currently, there are no battery tagging systems to check quality or tracking technology to monitor how batteries are used. This makes proper end-of-life disposal almost impossible. Without a clear national list of operators to ensure accountability, and without formal rules for recycling, the entire sector will continue to cause environmental damage and public health problems.

Reference:

- https://www.researchgate.net/publication/353244266_Life_Cycle_Analysis_of_Lead_Acid_Battery_used_in_Electric_Vehicles_3_Wheeler_in_Bangladesh#:~:text=Even%20industries%20prefer%20to%20buy,enough%20if%20not%20implemented%20properly.

- https://www.unep.org/resources/assessment/assessment-informal-used-lead-acid-battery-recycling-and-associated-impacts#:~:text=Based%20on%20these%20findings%2C%20informal,potential%20among%20the%20exposed%20population.

- https://smepprogramme.org/the-electric-three-wheeler-sector-in-bangladesh-regulating-for-growth-preventing-lead-pollution-and-unlocking-green-investment/#:~:text=Formalizing%20the%20electric%20three%2Dwheeler,registration%20and%20battery%20management%20tracking.

Transition Challenges to Lithium-Ion Technology

Lithium-ion batteries (Li-ion) are much better: they hold more energy and last much longer (about 4,000 cycles versus 300-600 for lead-acid). However, their significantly higher initial cost stops people from adopting them. Li-ion prices range from 56,000 to 200,000 BDT, while lead-acid costs just 12,000-23,000 BDT. Meanwhile, the short lifespan of current easy bike batteries (costing over 72,000 BDT but lasting only 8 to 11 months) pushes up operating costs and harms the millions of drivers and owners who rely on these vehicles for their income. Finally, the system for recycling Li-ion batteries is also not ready in Bangladesh. Information about recycling centers and how to process the batteries is very limited.

Reference:

- https://vibms.com/lithium-vs-lead-acid-batteries-higher-cost-worth/#:~:text=Choosing%20a%20battery%20often%20starts,costs%20and%20improve%20operational%20reliability.

- https://ipsbazarbd.com/product-category/battery/lithium-battery/#:~:text=The%20lithium%20battery%20price%20in,good%20options%20at%20various%20prices.

- https://manlybattery.com/lead-acid-vs-lithium-ion-battery-life/

- https://www.bssnews.net/news/281666#:~:text=The%20entire%20team%20presented%20research,technology%2Dbased%2C%20green%20Bangladesh.

- https://smepprogramme.org/project/solutions-to-informal-recycling-of-used-lead-acid-batteries/

6.4Economic

High Upfront Costs and Affordability Barriers

While EVs save money in the long run on running costs, the initial purchase price is simply too high for most Bangladeshi buyers when compared to standard petrol or diesel cars.

For four-wheeled vehicles, the issue is excessive import taxes and customs duties, which make EVs overpriced. Such as, EVs with large motors (over 175kW) are taxed as heavily as if they had a massive 3,500cc engine, leading to taxes as high as 200,000 BDT. Furthermore, unlike its neighbors, Bangladesh does not offer any subsidies to encourage the local manufacturing of electric vehicles.

The situation remains difficult even for more affordable three-wheelers. The cost of replacing batteries severely impacts the total expense of owning the vehicle. A new set of easy bike batteries costs 70,000-90,000 BDT but only lasts 1 to 1.5 years. These frequent, high replacement costs put a huge financial strain on drivers and owners who often operate with very little profit.

Reference:

- https://thefinancialexpress.com.bd/economy/bangladesh/transport-authority-proposes-tax-on-electric-vehicles-from-next-fiscal-1615692379

- https://www.thedailystar.net/business/news/high-registration-cost-taxes-slow-bangladeshs-shift-cleaner-evs-3773236

- https://www.alibaba.com/showroom/rickshaw-battery-bangladesh.html

- https://happymarsbd.com/product-category/rickshaw-battery/easy-bike-mishuk-rickshaw-battery-all/mishuk-battery/easy-bike-battery-all/#:~:text=Action%20Power%20200ah%20Easy%20Bike,%E0%A7%B3%2011%2C500%20%E0%A7%B3%2010%2C900

Limited Incentive Structures

The government has attempted to encourage EV adoption through tax breaks and import duty exemptions, but these measures don't quite match up to what other countries are doing. For example, India's FAME-II program offers significant subsidies, like up to 1.5 lakh BDT for four-wheeled vehicles and up to 40% of the purchase price for two-wheeled vehicles.

The environmental surcharge which is proposed on second vehicles, based on power rating, could also create an issue. This fee might actually discourage people from buying electric cars as a second vehicle. If the government doesn't remove this surcharge for EVs, the policy could unintentionally slow down the shift from gasoline cars to electric alternatives, which is what it's trying to promote.

Reference:

(1) https://heavyindustries.gov.in/en/fame-ii#:~:text=Faster%20Adoption%20&%20Manufacturing%20of%20Electric,scheme%20as%20a%20mass%20segment.

(2) https://www.autocarpro.in/news-national/electric-two-wheelers-set-to-get-costlier-fame-ii-subsidy-slashed-to-15-from-40-115147

(3) https://ecozaar.in/blog/india-ev-subsidy-guide-fame2/

Market Development Constraints

The Electric vehicles market is very sensitive to price, and high initial costs make it difficult for many to afford EVs. Also, there aren't many EV models available, which limits choices and keeps prices from becoming more competitive. Because Bangladesh relies on imports, costs are higher, making EVs less accessible, especially for those in rural or smaller urban areas.

Reference :

- https://crackplatooncsl.com/what-are-the-challenges-of-electric-vehicles-in-bangladesh/#:~:text=Get%20Clear%20Idea,to%20understand%20the%20full%20picture.

- https://www.researchgate.net/publication/390293199_Unleashing_the_Factors_Inducing_the_Adoption_of_Electric_Vehicles_EVs_in_Bangladesh_A_PLS-SEM_Approach

6.5Social & Safety Concerns

Accident Rates and Traffic Safety

Electric three-wheelers—of which there are an estimated 3 to 4 million—dominate Bangladesh's electric vehicle sector. However, the biggest challenge they pose is the high risk of accidents. The involvement of battery-powered rickshaws and easy bikes in road crashes is steadily increasing, reaching 16.56% of all accidents in 2024, up from 13% in 2019. This upward trend reflects the rapid and uncontrolled spread of these vehicles without any corresponding upgrades in safety rules or proper operator training.

The fatality data is also alarming. In 2024, three-wheeled vehicles accounted for 19.08% of road traffic deaths, making them the second-leading cause of fatalities, only behind motorcycles (at 35.76%). The total number of road traffic fatalities in 2024 was 8,543, with battery-powered vehicles making up a significant and growing portion of these deaths.

Reference :

- https://smepprogramme.org/the-electric-three-wheeler-sector-in-bangladesh-regulating-for-growth-preventing-lead-pollution-and-unlocking-green-investment/#:~:text=Electric%20three%2Dwheelers%20%E2%80%93%20EZ%20bikes,lead%20recycling%20practices%20are%20widespread.

- https://buymeacoffee.com/reactbd/bangladesh-electric-rickshaw-revolution-growth-challenges

- https://asiantransportobservatory.org/documents/63/Bangladesh_20231002.pdf

- https://www.newagebd.net/post/country/254496/8543-people-killed-in-road-accidents-in-bangladesh-in-2024-report#:~:text=Following%20motorcycle%2Drelated%20accidents%2C%20trucks,per%20cent%20in%20Chattogram%20city.

- https://en.prothomalo.com/bangladesh/m4kzejfpvx#:~:text=Bangladesh%20witnessed%207%2C294%20road%20accident,injuries%2C%20and%2039%20people%20missing.

Contributing Factors to Safety Concerns

One significant factor that raises accident risk is that many drivers are underage and haven't received proper training or licensing, leading to hazardous conditions on the road. Moreover, many drivers don't understand traffic rules or defensive driving techniques, which may lead to unpredictable behavior on the roads. Speed management is another issue, as slow-moving battery-powered vehicles mix with faster traffic on major roads without designated lanes or speed limits.

Mechanical problems are a big part of the safety issue. When things like the brakes don't work well or the suspension is worn out, the vehicle is much more likely to flip over, or rollover. This risk is even higher if the vehicle is carrying too much weight. Since there are no required vehicle inspections, these broken-down cars and trucks keep being used. That means everyone on the road—passengers and others—is facing greater danger.

Reference :

- https://www.researchgate.net/publication/375269582_Road_Accident_and_Safety_Issue_in_Bangladesh_A_Critical_Review

- https://www.researchgate.net/profile/Linnet-Barman/publication/387293503_Analysing_the_Key_Causes_of_Road_Accidents_in_Bangladesh/links/676d2757117f340ec3d913c2/Analysing-the-Key-Causes-of-Road-Accidents-in-Bangladesh.pdf

Public Awareness and Knowledge Gaps

One remarkable thing is that most of the people are not aware enough about the benefits of EVs. Potential buyers often have incorrect ideas about their performance, reliability, and maintenance needs. Even current EV users often don't know enough about how to handle batteries properly, charge them safely, and perform maintenance.

Reference:

- https://crackplatooncsl.com/what-challenges-are-ev-owners-currently-facing-in-bangladesh/

9. Challenges and Possible Solutions (Rakesh Biswas)

Addressing the challenges of battery-powered vehicles in Bangladesh requires well-rounded solutions that involve many stakeholders, balancing the need to get these vehicles on the road quickly with the importance of safety, sustainability, and economic practicality.

9.1 Energy Infrastructure Solutions

Grid Modernization and Load Management

Bangladesh needs a plan to handle the growing electricity needs of electric vehicles (EVs) without straining the current power grid. This means using smart grid technology and finding better ways to manage how much electricity is used. It's also vital to use the power sector's current capacity as much as possible to reduce energy waste. Utility companies should also explore how to make future charging systems more environmentally friendly by incorporating solar power.

Reference:

- https://ieefa.org/resources/faster-electric-car-rollout-bangladesh-demands-policy-support#:~:text=Additionally%2C%20the%20growing%20popularity%20of%20electric%20cars,unused%20capacity%20of%20Bangladesh's%20current%20power%20system.

- https://www.sciencedirect.com/science/article/pii/S2949948824000313#:~:text=Bangladesh%20is%20advancing%20toward%20creating%20paperless%20offices%2C,transparency%2C%20efficiency%2C%20and%20effectiveness%20in%20service%20delivery.

Accelerated Charging Infrastructure Development

To promote the growth of electric vehicle charging, SREDA and local power companies should work closely together. The government should aim high when setting goals for the number of charging stations, making sure to consider specific needs in cities, suburbs, and rural areas so that everyone has good access. The government can also work with private companies to build charging stations more quickly. By sharing the investment risks and using the private sector's efficiency, infrastructure development can be accelerated. The Bangladesh Energy Regulatory Commission should finalize the service charge structures to ensure that charging stations are profitable for operators while remaining affordable for consumers.

Charging corridors along highways should be prioritized to enable long-distance travel and reduce worries about running out of charge. Solar-powered charging stations are particularly promising for Bangladesh, using the country's abundant solar resources while reducing dependence on the grid and consumption of fossil fuels.

Reference:

- https://www.observerbd.com/news/444685#:~:text=According%20to%20the%20MoU%2C%20SR,of%20Bangladesh%2C%22%20he%20added.

- https://www.energytransitionbd.org/news/why-is-ev-infrastructure-investment-crucial-to-bangladesh%E2%80%99s-green-future%3F#:~:text=According%20to%20Goldman%20Sachs%20Research,%2C%20clean%2C%20and%20economically%20vibrant.

9.2 Regulatory Framework Solutions

Formalization and Official Recognition

Stakeholder workshops have resulted in eight policy recommendations, including officially regulating the electric three-wheeler sector, implementing battery tagging systems to ensure manufacturing quality, and establishing tracking technology to monitor batteries and manage their end-of-life. The Bangladesh Road Transport Authority (BRTA) should officially acknowledge three-wheelers, which would allow the Bangladesh Standards and Testing Institution (BSTI) to monitor the vehicles and their batteries. This would enable vehicle registration and battery tracking. Also, a temporary registration initiative should be introduced, allowing current three-wheeler operators to register without facing large fines. After this initiative, the government should ensure that all vehicles are registered and that those who aren't are held accountable.

Reference:

- https://www.dhakatribune.com/bangladesh/388489/tougher-policy-soon-to-regulate-battery-run

- https://www.tbsnews.net/bangladesh/e-rickshaws-be-introduced-uttara-dhanmondi-paltan-areas-august-1175821

- https://thefinancialexpress.com.bd/trade/move-to-legalise-electric-vehicles-1548302855

Safety Standards and Vehicle Certification

Comprehensive safety standards that cover structural integrity, braking systems, suspension quality, and electrical safety must be developed and enforced. Programs that require vehicles to be inspected, similar to those for regular cars, should be implemented to assess whether vehicles meet safety standards before they are registered, and regularly after that. Standardization protocols should be based on international best practices while also considering the specific conditions in Bangladesh. The highest levels of regulations should adhere to international protocols ratified by Bangladesh concerning the harmonization of motor vehicles as adopted by the United Nations.

Reference:

- https://gpilondon.com/publications/can-passing-new-laws-ensure-safer-roads-in-bangladesh

- https://www.tbsnews.net/bangladesh/transport/new-road-safety-law-works-curb-accidents-fatalities-1175626

- https://www.tbsnews.net/bangladesh/law-order/road-safety-law-stuck-drafting-despite-rising-fatalities-1223761

- https://moind.portal.gov.bd/sites/default/files/files/moind.portal.gov.bd/notices/d17e0566_13cb_486d_b77e_2d4063d09426/Automobile%20Industry%20Development%20Policy,%202020%20Draft.pdf

- https://legislativediv.portal.gov.bd/sites/default/files/files/legislativediv.portal.gov.bd/page/c0e8021e_5e8f_4ede_9050_f2faf351c2ad/24.%20Road%20Transport%20Act%2C%202018.pdf

Licensing and Training Requirements

A detailed licensing system must be created just for battery-powered vehicle drivers. This system should involve a knowledge test (written) and a skills test (practical driving). People should also use national IDs to check ages and keep children from driving these vehicles. Mandatory training programs should teach drivers about vehicle operation, road rules, safe driving (defensive techniques), battery risks, and emergency steps. By working with manufacturers and operators, we can build training facilities that offer high-quality lessons for everyone. The licensing should be tiered—meaning more rigorous standards are needed for drivers who use these vehicles for commercial use or carrying passengers.

Reference:

- https://www.truckbd71.com/2022/12/driving-licence-procedure-and-rules-2023.html#:~:text=Driving%20License%20Application%20Procedure%202023,participate%20in%20the%20written%20test.

- https://www.network.com.bd/resources/sourcing-procurement-services/bangladesh-driving-license-conversion/#:~:text=Take%20The%20Best%20Assistance%20For,practice%20it%20as%20a%20profession.

- http://bdlaws.minlaw.gov.bd/act-654/section-22065.html#:~:text=(%20Ordinance%20NO.%20LV%20OF%201983%20),a%20motor%20vehicle%20in%20any%20public%20place.

9.3 Battery Management and Circular Economy Solutions

To properly manage batteries and establish a circular economy for battery-powered vehicles in Bangladesh, it's essential to shift towards better-quality batteries. This can be achieved through new business approaches and microfinance options that encourage efficient battery return and recycling within the formal sector. "Battery-as-a-service" models can also help lower initial expenses while ensuring professional battery management and promoting safe disposal practices. Establishing formal recycling procedures, licensing and regulating recycling facilities, and implementing extended producer responsibility programs are crucial for creating a sustainable battery ecosystem. Moreover, integrating collection networks with vehicle service centers and charging stations, as well as using deposit-refund systems, can make it easier for users to return batteries and encourage responsible end-of-life management.

Reference:

- https://ric.gov.bd/call_for_proposal/how-can-challenges-in-sustainable-battery-disposal-be-addressed/#:~:text=In%20response%20to%20these%20challenges,hazardous%20materials%20are%20handled%20responsibly.

- https://www.thedailystar.net/tech-startup/news/battery-recycling-bangladesh-discounted-path-environmental-risk-3800611

9.4 Economic and Financial Solutions

Bangladesh needs smarter incentives to make electric vehicles (EVs) truly popular in the country. So the government should consider reducing import taxes for a while to encourage more people to buy EVs. They should also look at giving tax breaks to EV owners, similar to the system used in India. Waiving the fee for a second vehicle—if that second vehicle is electric—could also make it easier for people to switch from gas cars to EVs. Taxing vehicles differently based on how much they pollute could also make a difference, giving cleaner vehicles a price advantage. Taxing high-emission vehicles heavily while keeping taxes low on zero-emission EVs shifts the market towards greener choices without needing a lot of direct financial support. Giving tax breaks for ten years to companies that invest in building and putting together energy-efficient vehicles can attract manufacturers, creating jobs and reducing the need to import everything. Supporting local companies that make parts for these vehicles helps build a strong supply chain and improve technical skills.

Reference:

- https://crackplatooncsl.com/what-is-the-ev-policy-in-bangladesh/#:~:text=Vehicles%20powered%20by%20electric%20motors,more%20about%20Bangladesh's%20EV%20policy.

- https://lightcastlepartners.com/insights/2021/06/the-current-shape-of-ev-and-hybrid-vehicle-ecosystem-in-bangladesh/

- https://www.tbsnews.net/bangladesh/electric-vehicle-gets-incentive-package-local-manufacturing-1159751

11. Conclusion (Rakesh Biswas)

Battery-powered vehicles present both a big opportunity and a tough challenge for Bangladesh as it strives for sustainable transportation. With millions of electric three-wheelers already in use, the country has shown there's a real need for affordable transportation options. However, this rapid growth has happened without the necessary regulations, infrastructure, and safety measures. Issues like inadequate charging stations, unsafe battery recycling, affordability problems, and safety concerns paint a picture of an industry at a critical point. The promise of electric vehicles to help the environment depends on fixing these underlying problems. The current way batteries are managed leads to health hazards and environmental damage, undermining the potential benefits of electric vehicles. To solve this problem, Bangladesh needs to adopt comprehensive solutions that create a circular economy.

Bangladesh can reach its goals for lowering emissions and using more electric vehicles by tackling key challenges. This means improving how different groups work together, building new skills, and carefully overseeing the process. The suggested solutions provide a clear path for turning these obstacles into opportunities. These solutions include upgrading the power grid, building more charging stations, improving battery quality, establishing recycling programs, adjusting financial incentives, and setting up safety standards. The economic side of things also needs careful consideration. While battery-powered three-wheelers have done well because they're affordable, making four-wheelers more accessible means tackling the cost problems through incentives. Policy changes should boost demand through tax reforms and lower fees, while ensuring charging stations are easily available for long-distance trips.

Reference:

- https://www.thedailystar.net/tech-startup/news/battery-recycling-bangladesh-discounted-path-environmental-risk-3800611

- https://ric.gov.bd/sectors/energy/#:~:text=This%20lack%20of%20standard%20process,in%20battery%20recycling%20and%20reuse.

- https://www.thedailystar.net/tech-startup/news/battery-recycling-bangladesh-discounted-path-environmental-risk-3800611

- https://chargetech.net.au/electric-vehicle-charging-infrastructure/#:~:text=This%20accessibility%20eliminates%20the%20need,charging%20network%20in%20greater%20detail.

- https://crackplatooncsl.com/what-are-the-challenges-of-electric-vehicles-in-bangladesh/#:~:text=Get%20Clear%20Idea,to%20understand%20the%20full%20picture.

- https://crackplatooncsl.com/what-is-the-current-status-of-electric-vehicles-in-bangladesh/#:~:text=Lower%20Taxes%20and%20Duties:%20The,also%20supports%20clean%20transport%20growth.