Table-5: Framework for Managing and Reducing Challenges of Battery-Powered Electric Vehicles in Bangladesh

| Challenge Framework | Concept | Challenges |
| --- | --- | --- |
| Safety and Structural Management [1] | Vehicle frame design and braking system optimization to ensure passenger and driver safety through standardized specifications and regular inspections. | Establishing uniform safety standards and acceptance of rigorous vehicle inspection protocols; Poor braking and suspension systems causing fatal accidents; Structural weakness prone to rollover incidents. |
| Traffic and Congestion Control [1] | Regulation of E3W movement patterns and designated operational zones to reduce traffic congestion and minimize speed-related conflicts with other vehicles. | Implementing traffic coordination for slow-moving vehicles; Lack of designated lanes and stopping zones; Creating acceptable speed management protocols without reducing operational efficiency. |
| Driver Licensing and Age Verification [1] | Standardized driver qualification processes with age verification and proper training certification to ensure competent vehicle operation. | Establishing comprehensive licensing requirements and acceptance of digital verification systems; High prevalence of underage drivers; Difficulty in enforcement mechanisms. |
| Electrical Grid Integration and Load Management [1] | Demand-side management strategies and smart charging infrastructure to balance E3W energy consumption with national grid capacity. | Implementing controlled charging protocols and off-peak utilization; Uncontrolled overnight charging causing peak load shifts; Harmonic distortion affecting grid stability and power quality. |
| Battery Quality and Lifecycle Management [1] | Standardized battery specifications, proper maintenance protocols, and efficient recycling systems to extend battery lifespan and reduce environmental impact. | Ensuring adequate battery health monitoring and replacement timelines; Low battery lifecycle (9-12 months) increasing operational costs; Lead acid disposal contaminating soil and groundwater. |
| License Tracking and Vehicle Registration [1] | Comprehensive registration system and digital tracking of all operating E3Ws to ensure regulatory compliance and safety monitoring. | Establishing centralized database management and real-time monitoring systems; Lack of unified registration reducing government oversight; Difficulty identifying unregistered and substandard vehicles. |

References: [1] Shaha, N., Uddin, B., Khan, Z.R., et al. (2022). Affordable Electric Three-Wheeler in Bangladesh: Prospects, Challenges, and Sustainable Solutions. Sustainability, 15(1), 149. https://doi.org/10.3390/su15010149

Table 7: Summary of Detected Threats and Challenges in Battery-Powered Vehicle Sector in Bangladesh and Their Proposed Solutions

| Serial No. | Threats/Challenges | Proposed Solutions |
| --- | --- | --- |
| 01 | Safety Standards Incompatibility with Existing Vehicle Framework [1] | Adoption of internationally recognized safety standards including mandatory installation of Alpha frame design with reinforced structure, hydraulic braking systems, and seat belt mechanisms aligned with BRTA draft guidelines. |
| 02 | Poor Battery Management and Charging Infrastructure [1] | Implementation of centralized Battery Management Systems (BMS) with smart charging protocols; Establishment of regulated charging stations with proper ventilation systems and legal electricity connections; Introduction of battery swapping facilities to extend vehicle range and preserve battery health. |
| 03 | Inadequate Driver Training and Licensing Programs [1] | Development of structured driver licensing programs through BRTA with age verification requirements; Mandatory training on emergency response, traffic rules, and vehicle operation; Integration of digital identification systems and biometric verification for license tracking. |
| 04 | High Operational Costs and Grid Impact Expenses [2] | Implementation of demand-side management (DSM) strategies limiting charging to off-peak hours (10 PM-5 AM); Government subsidy mechanisms and custom duty exemptions on battery cells; Development of renewable energy-based charging infrastructure using solar and wind resources to reduce grid burden. |
| 05 | Limited Range and Battery Lifecycle Degradation [1] | Transition from lead-acid to Li-ion battery technology with 3-4 times longer lifespan; Integration of regenerative braking systems to recover kinetic energy; Adoption of proper battery maintenance protocols maintaining state of charge (SOC) between 40-50% to maximize battery longevity. |
| 06 | Environmental Threats and Battery Disposal Crisis [1] | Establishment of dedicated battery recycling facilities for both lead-acid and Li-ion batteries; Development of hazardous waste management protocols preventing soil and groundwater contamination; Implementation of extended producer responsibility (EPR) schemes requiring manufacturers to manage end-of-life batteries. |
| 07 | Traffic Congestion and Movement Regulation Issues [2] | Creation of dedicated lanes for electric three-wheelers with designated stopping zones; Implementation of IoT-based traffic management systems for real-time vehicle coordination; Development of movement restriction policies during peak traffic hours with incentive-based compliance mechanisms. |

References:

[1] Shaha, N., Uddin, B., Khan, Z.R., et al. (2022). Affordable Electric Three-Wheeler in Bangladesh: Prospects, Challenges, and Sustainable Solutions. Sustainability, 15(1), 149. https://doi.org/10.3390/su15010149

[2] Awal, M.R., Islam, A.N., & Khan, M.Z.R. (2019). Bangladesh power system peak demand shaving through demand side management of the battery operated easy bike load. Proceedings of the 4th International Conference on Electrical Information and Communication Technology (EICT), pp. 1–6. https://doi.org/10.1109/EICT48899.2019.9068796

Comprehensive Solutions for Battery-Powered Vehicle Challenges in Bangladesh: Summary

1. Safety and Structural Framework Solutions

Addressing the critical safety issues in battery-powered vehicles requires a multifaceted approach centered on design standardization and component quality. Bangladesh must mandate the adoption of the Alpha frame design with enhanced structural integrity through BRTA guidelines, implementing mandatory structural certification before vehicle registration and providing technical assistance to local manufacturers. The transition from mechanical to hydraulic disc braking systems should occur over a three-year phase-in period, supported by established testing laboratories certifying brake components and specialized technician training programs. Active Tilt Control (ATC) systems should be implemented to reduce rollover risk, with pilot programs in major cities (Dhaka, Chittagong, Khulna) offering subsidized technology transfer. Non-negotiable safety features including seat belts, safety harnesses, fire extinguishers, first-aid kits, and reflective markings must be mandatory from 2025, with comprehensive insurance requirements offering premium reductions for compliant vehicles.

2. Driver Licensing and Age Verification Solutions

The pervasive problem of underage and unlicensed drivers requires establishment of a centralized BRTA digital licensing database with biometric verification systems linked to national identification documents, preventing underage operation through tiered licensing (provisional at 18-21 years, permanent at 21+ years) with biennial renewal requirements. Structured driver training programs must be established in all districts with standardized curriculum covering vehicle operation, emergency response, traffic rules, and safety awareness, requiring minimum 40-hour training before license issuance. Special recruitment and training initiatives for female drivers should be launched with subsidized training and preferential loan schemes. Enforcement mechanisms must include mobile verification units at random checkpoints using portable scanners for instant verification, graduated penalty systems (warnings, fines, license suspension), and community engagement through incentive schemes for reporting violations.

3. Electrical Grid and Consumption Management Solutions

Mitigating the impact of electric vehicle charging on Bangladesh's power grid requires demand-side management strategies including time-of-use pricing offering 30-40% discounts for off-peak charging (10 PM-5 AM), mandatory timer circuits limiting charging to designated hours, and IoT-based smart meters enabling real-time consumption monitoring. Centralized high-capacity charging hubs (20-50 vehicles) should replace dispersed single-vehicle chargers, supported by software systems for optimal load distribution and Vehicle-to-Grid (V2G) technology allowing vehicle batteries to supply power during peak demand. Given Bangladesh's abundant solar resources (4-5 kWh/m2/day across 94% of territory) and coastal wind potential, renewable energy integration through solar-powered charging stations with net-metering systems should be prioritized, with government subsidies covering 50-70% of installation costs. These interventions can reduce peak load pressure and shift the timing advantage toward environmental sustainability.

4. Battery Quality and Lifecycle Management Solutions

Battery-related challenges must be addressed through progressive transition from lead-acid to lithium-ion technology, which offers 3-4 times longer lifecycle (2-3 hour charging versus 12-16 hours), reduced size and weight, and lower environmental impact. Comprehensive Battery Management Systems (BMS) should be implemented to regulate charging processes and prevent overcharging, with standardized specifications ensuring cell-level monitoring and current leakage detection. Proper maintenance protocols maintaining state of charge (SOC) between 40-50% can significantly extend battery longevity. Establishment of dedicated battery recycling facilities for both lead-acid (with existing companies like Rahim Afrooz and Panna Battery) and emerging Li-ion recycling infrastructure is critical, alongside extended producer responsibility schemes requiring manufacturers to manage end-of-life batteries, preventing contamination of soil, groundwater, and aquatic ecosystems.

5. Traffic Congestion and Movement Regulation Solutions

Managing traffic impacts requires creation of dedicated lanes for electric three-wheelers with designated stopping zones, reducing conflicts with faster-moving vehicles and improving traffic flow predictability. IoT-based traffic management systems enabling real-time vehicle coordination should be implemented, with movement restriction policies during peak hours supported by incentive-based compliance mechanisms rather than purely punitive approaches. Integration of E3Ws into broader urban mobility planning through coordination with public transport authorities can optimize route allocation and reduce overall congestion, while flexible operational hours (nighttime and off-peak deliveries) can distribute demand more evenly across the day.

6. Environmental Protection and Hazard Prevention Solutions

Preventing environmental hazards from improper battery disposal and uncontrolled charging requires stringent protocols including mandatory ventilation systems at charging stations, hazardous waste handling training for all operators, and regulatory oversight through BERC and BEPRC guidelines. Battery-swapping facilities should be established as sustainable alternatives to home charging, providing safe environments for battery exchange and enabling battery health monitoring. Extended producer responsibility schemes should require manufacturers to design batteries for efficient recycling and provide take-back programs, while awareness campaigns targeting vehicle owners and operators can promote proper disposal practices.

7. Economic Viability and Policy Support Solutions

Ensuring long-term economic viability requires government support through custom duty exemptions on battery cells, capital subsidies for charging infrastructure development, and preferential loan schemes for vehicle purchase through microfinance institutions and banks. Phased implementation allowing existing CNG operators to transition gradually through retrofitting programs or trade-in schemes can reduce resistance while maintaining livelihoods. Establishment of a dedicated E3W Development Board coordinating across BRTA, BERC, BEPRC, and local authorities can ensure coherent policy implementation and resolve jurisdictional conflicts. Regular stakeholder consultations with vehicle owners, drivers, mechanics, passengers, and manufacturers should inform policy adjustments as implementation progresses, ensuring solutions remain contextually appropriate and economically sustainable.