Exactly the way a thesis committee expects:
Problem → Methodology → Simulation → Outputs → Analysis → Validation → Results → Future Scope.

Complete Research Workflow (For Pre-Defence)

“Simulation-based Traffic Analysis on a Custom Road Network Using SUMO with EV Charging and Cost Modelling”

1. Problem Definition

Define the core research goals:

- Build a realistic custom road network from your field area.

- Simulate traffic flow, routes, and vehicle behaviors.

- Integrate electric vehicles (EVs) with charging stations placed at nodes.

- Derive travel costs, energy consumption and SOC (State of Charge-battery) drop.

- Perform decision-making / optimization using the above outputs.

2. Data Preparation Workflow

2.1 Build Custom Road Network

Tools: NETEDIT + SUMO
Tasks:

- Define nodes, edges, lanes, shapes.

- Configure junctions, traffic lights, and turning connections.

- Validate using:

- netcheck --net=Test1.net.xml

2.2 Define Routing (Demand Modeling)

- Create all possible routes from source → destination using SUMO edges.

- Store in Test1.rou.xml.

- Vehicles = EV + non-EV can be mixed.

2.3 Add Charging Stations

- One charging station per network node.

- Implemented in Test1.add.xml.

- Choose lane positions for realistic access.

2.4 Configure SUMO Simulation

SUMO configuration file includes:

- Network

- Routes

- Additional elements

- Battery + speed output

- Emission output (optional)

3. Simulation Workflow

Run simulation with all devices:

sumo-gui -c Test1.sumocfg \

--device.battery.probability 1 \

--battery-output battery.xml \

--emission-output emission.xml \

--fcd-output trajectory.xml

Outputs collected:

Battery/SOC

- Per timestep SOC, power consumption, charging events

Speed Profiles

- Instant speed per vehicle, lane-level flow

Travel Time

- start → end travel durations

Energy Consumption

- EV charge used on each route

- kWh cost

Queue Lengths

- At each intersection (esp. Node2 & Node6 since they have signals)

Waiting/Delay Times

- Measured from speed=0 events and stop durations

4. Travel Cost Matrix Construction

You already have a travel cost matrix — now integrate SUMO simulation metrics to refine it.

Travel Cost Components:

- Travel Time

- Energy Consumption

- Charging Cost

- Delay / Waiting Time

- Distance

- Average Speed

- Congestion/Queue Penalty

- SOC Drop

General Cost Function:

We can calibrate the weights using:

- statistical regression

- experimental values

5. Route Optimization or Decision-Making Model

After the travel-cost matrix is ready, apply optimization:

1. Shortest Path

Based on:

- time

- energy

- multi-objective (Pareto optimal)

2. EV-aware Routing

Constraints:

- SOC must not drop < 20%

- choose a charging station when needed

3. Traffic Light Optimization (optional)

Use queue length + delay simulations to adjust signal timings.

4. Clustering of routes (optional)

Using:

- K-Means / DBSCAN

- based on travel features

6. Visualization and Results

Produce research-grade graphs:

1. Speed vs Time graph

Per route or vehicle.

2. SOC Drop vs Distance graph

3. Travel Time Comparison for all Routes

Bar chart or radar chart.

4. Heatmap of Travel Cost Matrix

Use Python or MATLAB.

5. Queue length at junctions

Especially Node2 & Node6.

6. Charging Station Usage Statistics

- number of charging events

- waiting time at charging stations

- power consumed

7. Evaluation & Validation

pre-defence must show that:

The network is valid

(using SUMO netchecking)

Routing is realistic

(all edges & junctions used correctly)

Travel-cost matrix matches SUMO outputs

(e.g., travel time difference < 5%)

Energy consumption is consistent

(using SUMO battery model)

System supports scalability

(more vehicles, new routes, adaptive control)

8. Contributions (What You Have Achieved)

This is critical in a defence slide:

1. Created a full custom SUMO network manually.

2. Constructed complete route sets from Node1 → Node13.

3. Integrated EV charging infrastructure at all nodes.

4. Extracted real-time battery & speed metrics.

5. Built a complete travel-cost matrix.

6. Designed an optimization-ready simulation framework.

7. Provided a reproducible methodology for traffic/EV analysis.

9. What to Present in Pre-Defence Slides

Slide Structure:

- Introduction / Problem Statement

- Study Area & Custom Network

- Network Diagram (SUMO visualization)

- Routes & Travel Cost Matrix

- EV Model + Charging Layout

- Simulation Setup

- Outputs Collected

- Analysis & Results

- Optimization / Decision Model

- Conclusions & Future Work