A Simulation-Based Charging Station Recommendation Framework for Electric Vehicles in Khulna City Using SUMO and TraCI

Department of Computer Science & Engineering

Jashore University of Science and Technology (JUST)

Jashore,, Bangladesh

200112.cse@student.just.edu.bd

Abstract

The rapid growth of three wheeler electric vehicles – Easy Bikes, Electric-Rickshaws and Electric-Vans -all over the Khulna city of Bangladesh has great impact of unsupported charging demand on the urban power infrastructure. It isn’t prepared for a peak classification. This paper reflects simulation based charging station recommendation framework built on the Eclipse SUMO microscopic traffic simulator and its Python control interface and TraCI. A unique simulation with realistic battery parameter along with 100 vehicle-flows(34 Easy Bike, 33 E-Rickshaw, and 33 E-Van types across 8 routes) has created. It has 11 edge directed network and it’s duration 600 second . This simulation contains 10 bidirectional charging station(62 charging slots + 49 waiting slots). E-Rickshaws and E-Vans carry 5,760–7,200 Wh batteries. The proposed recommendation engine continuously observes vehicles charge-condition(SOC) and applies multi-valued decision model that evaluate charging station distance, slot occupancy and instant power load. Suggestive recommendations served per 30 seconds per vehicle. To maintain route flows and to ensure simulation stability – the structure works in suggestive decision making mode. Here proposed charging stations are evaluated based on vehicle’s current routing constraint and possibility and logged without hampering the predefined dynamicity designs. Experimental analysis of 1,169 recommendation events shows that the proposed framework is significantly better than the nearest-only-baseline strategy. The system  gains a recommendation probability rate of 90.1% compared to 66.0% for the baseline approach. It represents a 24.0 percentage-point improvement. Additionally, the framework achieves a congestion avoidance rate of 37.1%, where the baseline strategy achieves 0%. And a high-priority smart decision rate of 27.9%, compared to 0% under the baseline. During the simulation, the system recorded a peak charging power demand of 1,370 kW. Along with 40 vehicles charging simultaneously, resulting in a total energy consumption of 127.5 kWh in a 600-second session. The dataset consists of 33,721 vehicle-centric observation records and 1,169 charging recommendation events. It lays a broad foundation for further machine-learning-based load forecasting and smart grid planning studies in the context of the emerging EV ecosystem in Bangladesh.

Keywords—Electric Vehicles, Easy Bike, E-Rickshaw, Charging Recommendation, SUMO, TraCI, Smart Grid, Load Balancing, Bangladesh

I.  INTRODUCTION

The global adoption of electric vehicles (EVs) has accelerated as a strategy to prevent greenhouse gas (GHG) emissions from the transportation sector, which is responsible for approximately 38% of total global GHG production [1]. In Bangladesh, this change has taken an individual form: instead of personal passenger cars, battery-powered three-wheeled vehicles – locally known as Easy Bikes and E-Rickshaws – have become the main intercity transportation medium.

Khulna, the third largest city in Bangladesh and an important industrial hub, is a case in point. Thousands of electric three-wheelers ply the streets of Khulna every day, providing affordable, low-emission urban mobility and supporting the livelihoods of a significant driver population.

In spite of these facilities, EV charging in Khulna remains almost completely imbalanced. Drivers select charging stations based only on proximity and familiarity, which results in intense demand at popular stations during morning and evening rush hours. Transformers serving to those overloaded nodes - experience thermal stress, voltage drops, and failures. This results repair costs and disruption of driver service. In the meantime peripheral stations remain continuously unused, which highlights the infrastructure investment.Electric Vehicle load on the electricity grid is projected to reach 780 TWh by 2030 [3] globally. It makes intelligent charging coordination an urgent operational priority.

Previous work on EV charging management has mainly addressed advanced-economic contexts where vehicles are freely routable and have the advantage of inter-vehicle navigation.There are 2 limitations in the context of Khulna city which are absent in this literature : (i) Easy Bikes and Electric Rickshaws and Electric Vans follow specific corridor routes, as SUMO - the simulator used enforces one-way lane assignment; (ii)charging infrastructure is scattered and heterogeneous in terms of capacity, power rating and geographical distribution.This is a purpose-built simulation framework that realistically models the Khulna fleet and designs around it rather than ignoring routing limitations.

The paper represents a SUMO+TraCI simulation framework that models a heterogeneous fleet of continuous 100 vehicles(Easy Bikes, Electric Rickshaws, Electric Vans)on 8 road-routes. It monitors real time battery SOC and also issues per vehicle charging station recommendations through a multi criteria scoring models. This framework distinguishes between potential routing – when a directed network path exists. It also suggests only output when directional limitations prevent routing. In comparison with nearest only baseline, significant improvements are seen in potential recommendation rates, congestion avoidance and genius handling of high priority charging events.The contributions of this work are: (i) a calibrated SUMO simulation of Khulna city’s heterogeneous Electric Vehicle fleet with physics-based battery modeling on 3 vehicle classes; (ii) a multi-criteria  recommendation algorithm that clearly maintains SUMO directed routing; (iii) comparative evaluation with nearest only baseline using four performance metrics; (iv) a 33,721 recorded vehicle centric dataset.

II.  RELATED WORK

Planning of EV charging management research infrastructure, expands real time routing, timesteps. We study every thread to indicate our work position.

A.  Infrastructure Planning and Site Selection

Wang et al. [2] formulated charging station selection as a multi-objective optimization problem. It reduces travel time, grid pressure and cost through integer programming. Hussain et al. [6] applied analytical classification. It was Analytic Hierarchy Process(AHP). The methods combined with TOPSIS to rank candidate stations according to accessibility, demand density, and grid capacity.These methods inform long-term major decisions but do not indicate real-time dispatch across existing networks. Rahman et al. [1] surveyed adoption trends specific to Bangladesh, identifying coordination gaps, which are directly correlated to this study.

B.  Real-Time Routing and AI-Based Selection

Current research explored reinforcement learning & AI based frameworks for dynamics Electric Vehicle charging distribution. These methods target to balance the load at stations, to lessen traffic, decrease peak-demand through adaptive routing suggestion. Microscopic traffic simulation platforms like-SUMO has been widely implemented for traffic optimization. More implementation for signal control, energy aware mobility modeling. By the way fixed work aggregated real time suggestive recommendation of charging in to the heterogeneous EV fleet of South Asian cities.

C.  Load Forecasting and Dataset Generation

Siddiqui et al. [8] suggested DeepBoost—a stacked ensemble of LSTM, XGBoost, CatBoost, and Linear Regression for day ahead EV charging station load forecasting It improved the MAE by 9.4% and 32.7%, respectively, compared to individual CatBoost and XGBoost on the ACN dataset. Their work establishes - enriched feature engineering – station occupancy, depth of queue , SoC distribution, time of day . The work drives forecast accuracy. The dataset produced by our simulation accurately captures these features, making it directly usable as input for our charging station style models applied to the Khulna context.

D.  Research Gap

No previous work addresses: (i)simulation-based recommendation for corridor-limited Electric Vehicle fleet like Easy Bikes;(ii) a clear suggestive mode, which preserves simulation integrity ; or (iii) empirical load-balancing evaluation on a heterogeneous three-vehicle-type fleet with different battery capacities in a South Asian urban environment. This paper fills three gaps.

III.  SYSTEM ARCHITECTURE

Here the proposed structure consists of four components: the SUMO Traffic Simulation Engine, the TraCI Communication Layer, the Python based Charging Recommendation Engine and the Data Logging Module.

A.  Road Network and Traffic Simulation

The simulated network creates a representative model of a part of Khulna city. It consists of 11 directed edges(E0-E10) , bidirectional 8 designated routes(4 – forward, 4 - reverse) . Traffic Light System controls the intersection model with realistic phase timing. The simulation runs from t=0 to t=600s with a 0.1s step resolution, with data collected every integer second. Time-to-teleport is disabled to prevent artificial vehicle movement or routing. Lateral resolution is set to 0.8m to properly model the narrow lanes of Khulna city .

B.  Heterogeneous EV Fleet

To model Khulna’s real Electric Vehicle ecosystem , three vehicle classes with different battery configured: Easy Bikes(34 types), Electric Rickshaws(33 types), Electric Vans(33 types). Total 100 continuous flows across the network. From 2 to 81 vehicles per hour-launce vehicle through the 600 seconds simulation. Each vehicle has a SUMO battery device with specific values: 345 kg vehicle mass, 0.760 air drag coefficient, 0.0155 rolling drag, 81% propulsion efficiency, and 64% recovery efficiency. Table-I summarizes the three vehicle fleet classes. Significantly, the easy bikes carry 12,000 Wh batteries, whereas the e-rickshaws and e-vans are equipped with smaller batteries with 5,760–7,200 Wh batteries. This scenario reflectes the actual mixed-fleet composition deployed in Khulna. During simulation vehicles enter with different initial SoC levels(12.9% - 43.2%) , that reflects realistic field conditions.

TABLE I

Heterogeneous EV Fleet Configuration

| Parameter | Easy Bike | E-Rickshaw | E-Van |
| --- | --- | --- | --- |
| Number of types | 34 | 33 | 33 |
| Max. speed range (m/s) | 11.1–13.9 | 10.0–13.4 | 11.0–13.9 |
| Length / Width (m) | 2.5 / 1.5 | 2.3 / 1.0 | 2.4 / 1.3 |
| Max. battery capacity (Wh) | 12,000 | 5,760–7,200 | 5,760 |
| Initial SoC range (Wh) | 1,550–5,180 | 1,790–2,740 | 1,651–2,677 |
| Initial SoC (%) | 12.9–43.2% | 26.9–38.1% | 28.7–46.5% |
| Accel. range (m/s²) | 1.15–1.31 | 0.58–0.71 | 0.68–0.81 |
| Vehicle mass (kg) | 345 - 354 | 198 - 218 | 198 - 219 |
| Propulsion efficiency | 81% | 81% | 81% |

C.  Charging Station Infrastructure

Ten charging stations are embedded as SUMO parking areas across five road corridors (bidirectional), each offering separate charging and waiting sub-areas. Charging sub-areas are equipped with SUMO charging device parameters (chargingPower, chargingEfficiency) enabling native SUMO battery replenishment. Table II details the configuration. Total system capacity comprises 62 charging slots and 49 waiting slots. Station power ratings range from 25–50 kW at 95% efficiency.

TABLE II

Charging Station Configuration

| Station ID | Power (kW) | Charging Slots | Waiting Slots | Edge / Direction |
| --- | --- | --- | --- | --- |
| pa_2 / pa_2_rev | 30 / 30 | 6 / 6 | 6 / 3 | E0 / −E0 |
| pa_3 / pa_3_rev | 25 / 30 | 5 / 6 | 4 / 6 | E1 / −E1 |
| pa_6 / pa_6_rev | 30 / 30 | 6 / 6 | 6 / 3 | E5 / −E5 |
| pa_7 / pa_7_rev | 30 / 30 | 6 / 6 | 6 / 6 | E6 / −E6 |
| pa_8 / pa_8_rev | 25 / 50 | 5 / 10 | 3 / 6 | E7 / −E7 |
| System Total | — | 62 | 49 | — |

D.  TraCI Communication Layer

TraCI provides a socket-based API enabling per-step interaction with SUMO from Python. The controller retrieves battery parameters (device.battery.chargeLevel, device.battery.capacity, device.battery.totalEnergyConsumed), vehicle position, speed, current road ID, lane ID, and stop status at every integer second. It invokes traci.simulation.findRoute() to assess topological reachability—a critical step for the directional constraint—and traci.lane.getShape() to compute Euclidean station distances. Where rerouting is feasible and the vehicle's SoC falls below the 30% alert threshold, the controller calls traci.vehicle.setRoute() and traci.vehicle.setParkingAreaStop() to physically reroute the vehicle to its assigned parking area.

E.  Data Logging Module

Every recommendation event is logged as a 28-attribute CSV record: timestep, vehicle type, current edge, lane, speed, SoC, battery capacity, priority level, and full metadata for both the nearest and second-nearest stations (ID, distance, power rating, charging/waiting availability, occupancy rate), plus the recommended station name and textual reasoning string. The simulation generated 1,169 recommendation events. Additionally, a vehicle-centric ML-ready dataset captures 33,721 timestep-vehicle records with system-level features including total active vehicles, charging vehicles, average SoC, system power demand, and per-vehicle status and metrics.

IV.  CHARGING RECOMMENDATION MODEL

A.  Multi-Criteria Scoring Formulation

For a vehicle v requiring charging at simulation time t, the recommended station S* minimises a composite score over the set of N = 10 available stations:

S* = arg minᵢ  (w₁ Dᵢ + w₂ Qᵢ + w₃ Lᵢ)

where Dᵢ is the Euclidean distance from vehicle v to station sᵢ (m), Qᵢ is the number of vehicles currently in the waiting queue at sᵢ, and Lᵢ is the instantaneous charging power load at sᵢ (kW). Weighting factors w₁ = 0.5, w₂ = 0.3, w₃ = 0.2 were set to prioritise proximity while moderating congestion and load effects. The ranking is computed using Euclidean distance for computational efficiency at each 1-second polling interval; the top two candidates (nearest and second-nearest) then pass through the decision logic below.

B.  SoC Priority Classification

Each recommendation event is tagged with one of three priority levels based on the vehicle's current SoC: CRITICAL (SoC < 20%), HIGH (20% ≤ SoC < 30%), and MEDIUM (30% ≤ SoC < 50%). Priority-level tagging serves dual purposes: it modulates the urgency of console alerts during simulation (CRITICAL and HIGH events trigger verbose output) and constitutes a key feature in the generated ML dataset. The inspiration for explicit priority classification derives from Halder et al.'s energy demand score [7], which similarly quantifies urgency to enable intelligent resource prioritisation.

C.  Directional Constraint and Advisory Mode

A defining novelty of this framework is its explicit handling of SUMO's directional routing constraint. Easy Bikes and E-Rickshaws in Khulna operate on fixed corridor routes; SUMO enforces uni-directional lane assignments and cannot teleport vehicles to reverse edges. Consequently, the framework operates in two modes:

Rerouting Mode: When traci.simulation.findRoute() returns a valid directed path from the vehicle's current edge to the target station's edge with positive route length, the framework calls setRoute() and setParkingAreaStop() to physically redirect the vehicle. A return route to the vehicle's original destination is appended when feasible.

Advisory Mode: When no valid directed path exists—because the target station lies on an unreachable edge relative to the vehicle's current trajectory—the framework logs the best scoring station as an advisory recommendation without modifying the vehicle's route. This preserves simulation integrity and generates actionable data for a real-world notification system without forcing physically impossible manoeuvres.

D.  Four-Rule Decision Logic

Given the nearest station s₁ and second-nearest s₂, the recommendation algorithm applies the following rules in sequence:

Rule 1: If s₁ has a free charging slot → recommend s₁ (Rerouting Mode if path exists).

Rule 2: If s₁ is full for charging but s₂ has a free charging slot → recommend s₂.

Rule 3: If s₂ also lacks charging slots but s₂ has waiting capacity → recommend s₂ (waiting queue).

Rule 4: If both s₁ and s₂ are fully saturated → recommend s₁ (Advisory Mode, flag for retry).

A 30-second per-vehicle cooldown prevents recommendation churn. Queue management employs FIFO promotion: when a charging vehicle departs, the next waiting vehicle is automatically promoted to a charging slot via traci.vehicle.resume() and a new setParkingAreaStop() call, maintaining orderly slot throughput without external intervention.

V.  SIMULATION SETUP AND PARAMETERS

Table III summarises the complete simulation parameter set. The SUMO configuration (Test1.sumocfg) specifies the road network (Test1_with_TLS.net.xml), vehicle flows (Test1.rou.xml), and charging infrastructure (Test1.chargingstations.add.xml). Battery output, charging station output, trip information, and summary statistics are collected via SUMO's native output facility. The Python controller (SmartChargingStationMonitor) connects via TraCI, iterates at 1-second integer steps, and exports recommendations and ML-ready datasets to timestamped CSV files upon simulation completion at t = 600 s.

TABLE III

Key Simulation Parameters

| Parameter | Value |
| --- | --- |
| Simulation duration | 600 s |
| Step length (SUMO) | 0.1 s |
| Data collection interval | 1 s (integer steps) |
| Total vehicle flows | 100 (34 Easy Bike + 33 E-Rickshaw + 33 E-Van) |
| Vehicles per hour per flow | 2–81 (varies by route) |
| Number of routes | 8 (4 forward, 4 reverse) |
| Battery capacity range | 5,760–12,000 Wh (varies by vehicle class) |
| Propulsion efficiency | 81% |
| Recuperation efficiency | 64% |
| SoC recommendation threshold | < 50% |
| SoC alert / rerouting threshold | < 30% |
| Charging target SoC | 70% |
| Recommendation cooldown | 30 s per vehicle |
| Min / Max charge duration | 30 s / 300 s |
| Charging station power range | 25–50 kW |
| Charging efficiency | 95% |
| Scoring weights (w₁, w₂, w₃) | 0.5, 0.3, 0.2 |
| Total charging stations | 10 (5 corridors × 2 directions) |
| Total charging slots | 62 |
| Total waiting slots | 49 |
| Lateral resolution | 0.8 m |
| Time-to-teleport | Disabled (−1) |

VI.  RESULTS AND DISCUSSION

A.  System Load and Charging Statistics

The simulation concluded at t = 600 s, generating 33,721 vehicle-centric records (one per vehicle per second) and 1,169 charging recommendation events. The system recorded a peak instantaneous charging power of 1,370 kW with 40 vehicles simultaneously charging—reflecting near-saturation of the available 62 charging slots during peak demand periods. Average charging power across the simulation was 963.7 kW, and the total energy consumed for charging was 127.5 kWh. These load metrics demonstrate that the network experiences significant charging demand, making intelligent station selection essential to prevent localised transformer overload.

B.  Recommendation Volume and Priority Distribution

Table IV reports the priority-level breakdown of the 1,169 recommendation events. HIGH-priority events (SoC 20%–30%) dominated at 42.7%, followed by MEDIUM (34.6%) and CRITICAL (22.7%). The mean SoC at recommendation time was 27.7% (σ = 8.5%), with a minimum of 12.0% and a maximum of 46.5%, confirming that the 50% trigger activates recommendations with sufficient battery margin. The mean Euclidean distance to the nearest station at recommendation time was 84.3 m (σ = 62.0 m, min = 0.5 m, max = 256.6 m), indicating adequate station density for the modelled fleet.

TABLE IV

Recommendation Priority Distribution

| Priority | SoC Range | Count | Share |
| --- | --- | --- | --- |
| CRITICAL | < 20% | 265 | 22.7% |
| HIGH | 20%–30% | 499 | 42.7% |
| MEDIUM | 30%–50% | 405 | 34.6% |
| Total | — | 1,169 | 100% |

C.  Recommendation Outcome Analysis

Table V presents the distribution of recommendation outcomes. The dominant outcome—59.0% (689 events)—was routing to the nearest station when charging slots were available. In 24.1% of cases (282 events), the nearest station was fully occupied and vehicles were successfully directed to the second-nearest station with available capacity, demonstrating genuine load diversion. Waiting slot overflow management handled 7.7% (90 events), while 9.2% (108 events) triggered Advisory Mode where both stations were saturated.

TABLE V

Recommendation Outcome Analysis

| Recommendation Reason | Count | Share |
| --- | --- | --- |
| Nearest with charging slots (Rule 1) | 689 | 59.0% |
| 2nd nearest — slots available (Rule 2) | 282 | 24.1% |
| Nearest — waiting slots only (Rule 3) | 90 | 7.7% |
| Nearest — all busy, advisory only (Rule 4) | 108 | 9.2% |

D.  Station Utilisation Distribution

Station pa_8 received the highest recommendation share (32.4% of all events), reflecting its large slot capacity (5 charging + 3 waiting forward, 10 + 6 reverse) and up to 50 kW power rating combined with a central network position. Stations pa_7 and pa_3_rev ranked second and third at 14.7% and 14.0% respectively. Critically, all ten stations received at least 1.5% of recommendations, confirming that the multi-criteria scoring model distributes demand broadly.

E.  Comparative Evaluation: Proposed vs. Nearest-Only Baseline

To quantify the framework's benefit, we define a nearest-only baseline that always recommends the Euclidean nearest station without considering occupancy, queue, or load. Table VI reports four performance metrics computed from the 1,169-event dataset.

TABLE VI

Performance Comparison: Proposed vs. Nearest-Only Baseline

| Metric | Baseline | Proposed | Improvement |
| --- | --- | --- | --- |
| Feasible Recommendation Rate | 66.0% | 90.1% | +24.0 pp |
| Average Distance (m) | 84.3 | 100.1 | +15.8 m |
| Congestion Avoidance Rate | 0.0% | 37.1% | +37.1 pp |
| High-Priority Smart Rate | 0.0% | 27.9% | +27.9 pp |

The Feasible Recommendation Rate (FRR) measures the fraction of events where the recommended station had at least one slot available. The proposed framework achieves FRR = 90.1% versus 66.0% for the baseline—a 24.0 pp improvement—because it evaluates occupancy and diverts vehicles to alternatives. The Congestion Avoidance Rate (CAR) measures diversion away from a congested nearest station; the baseline achieves CAR = 0% while the proposed achieves CAR = 37.1%, quantifying load-balancing contribution.

The High-Priority Smart Decision Rate (HPSDR) measures the fraction of CRITICAL- or HIGH-priority events (SoC < 30%) where the framework diverted the vehicle to a second-nearest station with better availability. The proposed framework achieves HPSDR = 27.9% (vs. 0%), demonstrating intelligent routing when battery urgency is highest.

Average distance increases from 84.3 m to 100.1 m (+15.8 m)—an accepted trade-off for directing vehicles to stations with capacity. This distance increase is negligible given the remaining range at median SoC of 27.7%.

F.  Limitations and Future Work

The 600-second simulation window covers a simplified network. Full-scale deployment requires GPS-trace calibrated OD matrices and heterogeneous charging profiles from field surveys. The Advisory Mode (9.2% of events) could be reduced by increasing station density or adding a third-nearest candidate. Future work will integrate dynamic electricity pricing and renewable energy at station nodes.

VII.  IMPLICATIONS FOR BANGLADESH

The 37.1% Congestion Avoidance Rate implies that roughly one in three charging events that would otherwise overload a popular station can be redistributed—reducing peak transformer loading and extending equipment lifespan. The generated datasets (33,721 vehicle-centric + 1,169 recommendation events) provide precisely the feature space required by DeepBoost [8] for day-ahead load forecasting.

The per-station recommendation distribution reveals that pa_8 and pa_7 corridors absorb disproportionate demand—directly actionable information for capacity expansion. The open SUMO+Python architecture enables city planners to test alternative station placements by modifying the XML configuration without specialised optimisation expertise.

VIII.  CONCLUSION

This paper presented a SUMO+TraCI simulation-based charging station recommendation framework for Khulna city's heterogeneous EV fleet. A 100-flow simulation across three vehicle classes with varying battery capacities (12,000 Wh for Easy Bikes; 5,760–7,200 Wh for E-Rickshaws and E-Vans), eight routes, and ten bidirectional stations generated 1,169 recommendation events over 600 seconds, recording peak charging power of 1,370 kW with 40 vehicles simultaneously charging and consuming 127.5 kWh total energy.

The principal contributions are: (1) a physics-calibrated SUMO fleet model with heterogeneous battery configurations across three vehicle classes; (2) a multi-criteria recommendation algorithm with Rerouting/Advisory duality; (3) comparative evaluation demonstrating +24.0 pp Feasible Recommendation Rate, +37.1 pp Congestion Avoidance Rate, and +27.9 pp High-Priority Smart Decision Rate versus a nearest-only baseline; and (4) two complementary datasets (33,721 vehicle-centric records + 1,169 recommendation events) directly compatible with ML load forecasting.

Future work will extend to Khulna's full road graph with GPS-trace calibration, incorporate dynamic pricing, model renewable energy at stations, and evaluate demand-response scheduling aligned with Bangladesh's National Energy Policy.

REFERENCES

[1] M. A. Rahman, S. Islam, and A. H. Khan, "Electric vehicle adoption trends in Bangladesh: Opportunities and challenges," IEEE Access, vol. 9, pp. 145632–145645, 2021.

[2] D. Wang, J. Guan, T. Wu, G. Zheng, and Y. Hu, "Intelligent charging station selection for electric vehicles using multi-objective optimization," IEEE Trans. Intell. Transport. Syst., vol. 23, no. 8, pp. 11234–11245, Aug. 2022.

[3] C. Pascual, Global EV Outlook 2022, IEA, Paris, France, 2022. [Online]. Available: https://www.iea.org/reports/global-ev-outlook-2022

[4] S. Chen, Y. Tong, and W. Hu, "TraCI-based real-time traffic control in SUMO simulation," J. Traffic Transport. Eng., vol. 7, no. 3, pp. 315–328, 2020.

[5] X. Zhang, Q. Wang, and L. Li, "Load balancing strategies for electric vehicle charging stations using reinforcement learning," IEEE Trans. Smart Grid, vol. 13, no. 4, pp. 3124–3135, Jul. 2022.

[6] A. Hussain, V. H. Bui, and H. M. Kim, "A multi-criteria decision-making approach for electric vehicle charging station location selection," Energies, vol. 13, no. 23, p. 6322, 2020.

[7] M. Halder, A. Adhikary, S.-B. Park, and C. S. Hong, "An artificial intelligence framework for dynamic selection and resource allocation for EVs in vehicular networks," in Proc. IEEE/IFIP NOMS 2024, Seoul, South Korea, May 2024, DOI: 10.1109/NOMS59830.2024.10574932.

[8] J. Siddiqui, U. Ahmed, A. Amin, T. Alharbi, A. Alharbi, I. Aziz, A. R. Khan, and A. Mahmood, "Electric vehicle charging station load forecasting with an integrated DeepBoost approach," Alexandria Eng. J., vol. 116, pp. 331–341, Jan. 2025.

[9] M. R. Islam, H. Lu, J. Hossain, and L. Li, "A review on energy management strategies for electric vehicles in developing countries: A case study of Bangladesh," Renewable Sustain. Energy Rev., vol. 141, p. 110803, 2021.

[10] P. A. Lopez et al., "Microscopic traffic simulation using SUMO," in Proc. 21st Int. Conf. Intell. Transport. Syst. (ITSC), Maui, HI, USA, 2018, pp. 2575–2582.