📊 Complete Output Files Parameter Guide

1. BATTERY_DATA CSV/Excel Sheet

This contains timestep-by-timestep battery information for every electric vehicle.

Column-by-Column Breakdown:

| Column Name | Unit | Data Type | Meaning | Example Value | Interpretation |
| --- | --- | --- | --- | --- | --- |
| timestep_sec | seconds (s) | Float | Simulation time when data was recorded | 125.0 | At 125 seconds into simulation |
| vehicle_id | - | String | Unique vehicle identifier | "easybike_01" | The first easy bike |
| vehicle_type | - | String | Type of vehicle | "easyBike" | Easy bike category |
| actualBatteryCapacity_Wh | Watt-hours (Wh) | Float | Current battery charge remaining | 17442.75 | Battery has 17.44 kWh remaining |
| maximumBatteryCapacity_Wh | Watt-hours (Wh) | Float | Total battery capacity (100% charged) | 35000.00 | Full battery is 35 kWh |
| battery_soc_percent | Percentage (%) | Float | State of Charge percentage | 49.84 | Battery is 49.84% full |
| totalEnergyConsumed_Wh | Watt-hours (Wh) | Float | Cumulative energy used since departure | 71.13 | Used 71.13 Wh total |
| totalEnergyRegenerated_Wh | Watt-hours (Wh) | Float | Cumulative energy recovered through braking | 13.89 | Recovered 13.89 Wh |
| netEnergyUsed_Wh | Watt-hours (Wh) | Float | Actual energy consumed (consumed - regenerated) | 57.24 | Net consumption: 71.13 - 13.89 |
| chargingStationId | - | String | ID of charging station (if charging) | "fast_charger_n2" or "NULL" | Currently charging at this station, or not charging |
| speed_ms | meters/second (m/s) | Float | Instantaneous speed | 4.51 | Moving at 4.51 m/s |
| speed_kmh | kilometers/hour (km/h) | Float | Instantaneous speed in km/h | 16.2 | Moving at 16.2 km/h (4.51 × 3.6) |
| x_position_m | meters (m) | Float | X-coordinate in network | -80.33 | Position on X-axis |
| y_position_m | meters (m) | Float | Y-coordinate in network | 409.39 | Position on Y-axis |
| lane | - | String | Current lane identifier | "E0_0" | On edge E0, lane 0 |
| distance_traveled_m | meters (m) | Float | Total distance traveled since departure | 820.84 | Traveled 820.84 meters |
| waiting_time_sec | seconds (s) | Float | Time spent waiting (speed < 0.1 m/s) | 5.2 | Waited 5.2 seconds (traffic/lights) |

Calculated Formulas:

battery_soc_percent = (actualBatteryCapacity_Wh / maximumBatteryCapacity_Wh) × 100

netEnergyUsed_Wh = totalEnergyConsumed_Wh - totalEnergyRegenerated_Wh

speed_kmh = speed_ms × 3.6

distance_traveled_km = distance_traveled_m / 1000

Example Row Interpretation:

timestep_sec,vehicle_id,vehicle_type,actualBatteryCapacity_Wh,maximumBatteryCapacity_Wh,battery_soc_percent,totalEnergyConsumed_Wh,totalEnergyRegenerated_Wh,netEnergyUsed_Wh,chargingStationId,speed_ms,speed_kmh,x_position_m,y_position_m,lane,distance_traveled_m,waiting_time_sec

125.0,erickshaw_flow_r1.0,eRickshaw,17442.75,35000.00,49.84,71.13,13.89,57.24,NULL,4.51,16.2,-80.33,409.39,E0_0,820.84,0.0

Reading: At 125 seconds, e-rickshaw "erickshaw_flow_r1.0" has:

- 49.84% battery remaining (17.44 kWh out of 35 kWh)

- Used 71.13 Wh, recovered 13.89 Wh (net: 57.24 Wh)

- Not charging (NULL)

- Moving at 16.2 km/h

- At position (-80.33, 409.39) on lane E0_0

- Traveled 820.84 meters

- Not waiting

2. BATTERY_SUMMARY CSV/Excel Sheet

This contains one row per vehicle with trip-level summaries.

| Column Name | Unit | Data Type | Meaning | Example | Interpretation |
| --- | --- | --- | --- | --- | --- |
| Vehicle_ID | - | String | Vehicle identifier | "easybike_01" | First easy bike |
| Vehicle_Type | - | String | Type of vehicle | "easyBike" | Category |
| Total_Energy_Consumed_Wh | Watt-hours (Wh) | Float | Total gross energy used | 450.25 | Consumed 450.25 Wh during entire trip |
| Total_Energy_Regenerated_Wh | Watt-hours (Wh) | Float | Total energy recovered | 68.40 | Recovered 68.40 Wh through braking |
| Net_Energy_Used_Wh | Watt-hours (Wh) | Float | Actual energy depleted from battery | 381.85 | Net consumption (450.25 - 68.40) |
| Final_SoC_Percent | Percentage (%) | Float | Battery level at end of trip | 42.15 | Ended with 42.15% battery |
| Total_Distance_m | meters (m) | Float | Total distance traveled | 3044.00 | Traveled 3,044 meters |
| Total_Distance_km | kilometers (km) | Float | Distance in kilometers | 3.044 | 3.044 kilometers |
| Avg_Speed_kmh | kilometers/hour (km/h) | Float | Average speed throughout trip | 18.5 | Average 18.5 km/h |
| Energy_Efficiency_Wh_per_km | Watt-hours per kilometer (Wh/km) | Float | Energy consumption rate | 125.4 | Used 125.4 Wh per km traveled |
| Regeneration_Efficiency_Percent | Percentage (%) | Float | Percentage of consumed energy recovered | 15.19 | Recovered 15.19% of used energy |

Key Formulas:

Net_Energy_Used_Wh = Total_Energy_Consumed_Wh - Total_Energy_Regenerated_Wh

Total_Distance_km = Total_Distance_m / 1000

Energy_Efficiency_Wh_per_km = Net_Energy_Used_Wh / Total_Distance_km

Regeneration_Efficiency_Percent = (Total_Energy_Regenerated_Wh / Total_Energy_Consumed_Wh) × 100

Example Row:

Vehicle_ID,Vehicle_Type,Total_Energy_Consumed_Wh,Total_Energy_Regenerated_Wh,Net_Energy_Used_Wh,Final_SoC_Percent,Total_Distance_m,Total_Distance_km,Avg_Speed_kmh,Energy_Efficiency_Wh_per_km,Regeneration_Efficiency_Percent

easybike_01,easyBike,450.25,68.40,381.85,42.15,3044.00,3.044,18.5,125.4,15.19

Reading: Easy bike 01:

- Used 381.85 Wh net (after regeneration)

- Traveled 3.044 km

- Efficiency: 125.4 Wh/km

- Recovered 15.19% of energy through braking

- Ended trip with 42.15% battery

3. REALTIME_DATA CSV/Excel Sheet

Contains position and speed data for all vehicles at each timestep.

| Column Name | Unit | Data Type | Meaning | Example | Interpretation |
| --- | --- | --- | --- | --- | --- |
| timestep_sec | seconds (s) | Float | Simulation time | 50.0 | At 50 seconds |
| vehicle_id | - | String | Vehicle identifier | "evan_02" | Electric van 02 |
| vehicle_type | - | String | Type | "eVan" | Van category |
| speed_ms | meters/second (m/s) | Float | Speed | 12.5 | 12.5 m/s |
| speed_kmh | kilometers/hour (km/h) | Float | Speed in km/h | 45.0 | 45 km/h |
| x_position_m | meters (m) | Float | X coordinate | -250.45 | X position |
| y_position_m | meters (m) | Float | Y coordinate | 380.22 | Y position |
| lane | - | String | Current lane | "E1_0" | Edge E1, lane 0 |
| distance_traveled_m | meters (m) | Float | Cumulative distance | 1250.5 | Traveled 1250.5 m |
| waiting_time_sec | seconds (s) | Float | Waiting duration | 0.0 | Not waiting |

Usage:

- Track vehicle movements over time

- Create trajectory plots (x vs y)

- Analyze traffic flow patterns

- Identify congestion (low speeds, high waiting times)

4. TRIP_SUMMARY CSV/Excel Sheet

One row per vehicle with basic trip statistics.

| Column Name | Unit | Data Type | Meaning | Example | Interpretation |
| --- | --- | --- | --- | --- | --- |
| vehicle_id | - | String | Vehicle ID | "erickshaw_03" | E-rickshaw 03 |
| vehicle_type | - | String | Type | "eRickshaw" | Rickshaw category |
| depart_time | seconds (s) | Float | When vehicle entered simulation | 15.0 | Departed at 15 seconds |
| max_speed_kmh | kilometers/hour (km/h) | Float | Highest speed reached | 24.5 | Max speed: 24.5 km/h |
| total_waiting_time | seconds (s) | Float | Total time spent waiting | 45.3 | Waited 45.3 seconds total |
| final_distance | meters (m) | Float | Total distance at end | 2850.0 | Traveled 2850 meters |

Calculated Metrics You Can Derive:

travel_time = arrival_time - depart_time

average_speed = final_distance / travel_time

waiting_percentage = (total_waiting_time / travel_time) × 100

5. CHARGING_EVENTS CSV/Excel Sheet

Records every instance a vehicle charges.

| Column Name | Unit | Data Type | Meaning | Example | Interpretation |
| --- | --- | --- | --- | --- | --- |
| timestep | seconds (s) | Float | When charging occurred | 85.0 | At 85 seconds |
| vehicle_id | - | String | Which vehicle | "easybike_05" | Easy bike 05 |
| vehicle_type | - | String | Type | "easyBike" | Bike |
| charging_station | - | String | Station ID | "fast_charger_n2" | Fast charger at N2 |
| battery_soc_percent | Percentage (%) | Float | Battery level when charging | 25.4 | Started charging at 25.4% |

Usage:

- Count charging visits per station

- Identify low-battery patterns (vehicles that charge at low SoC)

- Analyze charging station usage

- Optimize station placement

6. CHARGING_SUMMARY CSV/Excel Sheet

Aggregated charging statistics by station.

| Column Name | Unit | Data Type | Meaning | Example | Interpretation |
| --- | --- | --- | --- | --- | --- |
| Charging_Station | - | String | Station identifier | "fast_charger_n2" | Fast charger at Node 2 |
| Total_Visits | count | Integer | Number of charging events | 45 | 45 vehicles charged here |
| Avg_SoC_at_Charge | Percentage (%) | Float | Average battery level when charging | 28.6 | Vehicles typically arrive at 28.6% |

Analysis:

- Most used stations (highest Total_Visits)

- Emergency charging (low Avg_SoC_at_Charge)

- Planned charging (high Avg_SoC_at_Charge)

7. VEHICLE_TYPE_STATS CSV/Excel Sheet

Aggregated statistics by vehicle type.

| Column Name | Unit | Data Type | Meaning | Example | Interpretation |
| --- | --- | --- | --- | --- | --- |
| Vehicle_Type | - | String | Type category | "easyBike" | Easy bikes |
| Count | number | Integer | Total vehicles of this type | 120 | 120 easy bikes in simulation |
| Avg_Speed_kmh | kilometers/hour (km/h) | Float | Average speed across all vehicles | 22.3 | Average 22.3 km/h |
| Max_Speed_kmh | kilometers/hour (km/h) | Float | Highest speed reached by any vehicle | 68.5 | One reached 68.5 km/h |
| Max_Distance_m | meters (m) | Float | Longest distance traveled | 3800.0 | One traveled 3800 m |
| Total_Waiting_Time_sec | seconds (s) | Float | Sum of all waiting time | 2450.0 | All bikes waited 2450s combined |

For Your Simulation:

Expected vehicle types:

- easyBike (60% of traffic)

- eRickshaw (25% of traffic)

- eVan (15% of traffic)

8. OVERALL_STATISTICS CSV/Excel Sheet

High-level simulation summary.

| Metric | Value | Unit | Meaning |
| --- | --- | --- | --- |
| Total Vehicles | 156 | count | Total vehicles simulated |
| Total Energy Consumed | 125.45 | kWh | Total gross energy used |
| Total Energy Regenerated | 18.75 | kWh | Total energy recovered |
| Net Energy Used | 106.70 | kWh | Actual energy depleted |
| Regeneration Efficiency | 14.94 | % | Percentage recovered |
| Simulation Duration | 200 | seconds | Total simulation time |
| Average Speed | 20.5 | km/h | Average across all vehicles |
| Total Distance Traveled | 245.8 | km | Cumulative distance |

Key Performance Indicators (KPIs):

Average Energy per Vehicle = Total Energy / Total Vehicles

Average Distance per Vehicle = Total Distance / Total Vehicles

Fleet Efficiency = Total Distance / Net Energy Used (km/kWh)

📐 Unit Conversion Reference

Energy Units:

1 Wh = 1 Watt-hour

1 kWh = 1000 Wh

1 kWh = 3.6 MJ (Megajoules)

Speed Units:

1 m/s = 3.6 km/h

1 km/h = 0.278 m/s

13.89 m/s = 50 km/h

Distance Units:

1 km = 1000 m

1 m = 0.001 km

Power Units:

1 W = 1 Joule/second

1 kW = 1000 W

50 kW charger = 50,000 W

Time Units:

1 minute = 60 seconds

1 hour = 3600 seconds

🔬 Example Analysis Calculations

1. Energy Consumption Rate:

From Battery_Summary:

Net_Energy_Used_Wh = 381.85 Wh

Total_Distance_km = 3.044 km

Energy Rate = 381.85 / 3.044 = 125.4 Wh/km

2. Range Remaining:

From Battery_Data (latest row):

actualBatteryCapacity_Wh = 17442.75 Wh

Energy_Efficiency = 125.4 Wh/km

Range = 17442.75 / 125.4 = 139.1 km remaining

3. Charging Time Estimate:

Battery deficit = 35000 - 17442.75 = 17557.25 Wh = 17.56 kWh

Charger power = 50 kW

Efficiency = 0.92

Charging time = (17.56 / 50) / 0.92 = 0.382 hours = 23 minutes

4. Trip Duration:

From Trip_Summary:

final_distance = 3044 m

average_speed = 18.5 km/h = 5.14 m/s

Travel time = 3044 / 5.14 = 592 seconds = 9.87 minutes

5. Regenerative Braking Effectiveness:

From Battery_Summary:

Total_Energy_Consumed = 450.25 Wh

Total_Energy_Regenerated = 68.40 Wh

Regen% = (68.40 / 450.25) × 100 = 15.19%

🎯 Data Quality Checks

Valid Ranges (Bangladesh Context):

| Parameter | Expected Range | Your Values |
| --- | --- | --- |
| easyBike max speed | 60-70 km/h | maxSpeed="19.44" m/s = 70 km/h ✓ |
| eRickshaw max speed | 20-25 km/h | maxSpeed="6.94" m/s = 25 km/h ✓ |
| eVan max speed | 40-50 km/h | maxSpeed="13.89" m/s = 50 km/h ✓ |
| Battery SoC | 0-100% | Check: Should never exceed 100% |
| Energy regenerated | < Energy consumed | Always: regen < consumed |
| Speed | ≥ 0 | Never negative |

📊 Interpreting Specific Values

Battery SoC Percentage:

- 90-100%: Fully charged, ready for long trip

- 50-90%: Healthy range, no charging needed

- 20-50%: Consider charging soon

- 10-20%: Should charge (yellow zone)

- 0-10%: Critical, must charge (red zone)

Energy Efficiency (Wh/km):

- < 100 Wh/km: Excellent (light vehicle, efficient driving)

- 100-150 Wh/km: Good (typical for e-bikes, e-rickshaws)

- 150-250 Wh/km: Average (heavier vehicles, urban driving)

- > 250 Wh/km: Poor (aggressive driving, heavy loads)

Regeneration Efficiency (%):

- < 5%: Minimal braking, highway driving

- 5-15%: Typical urban driving

- 15-25%: Good, lots of stop-and-go traffic

- > 25%: Excellent, very efficient braking recovery

Waiting Time:

- 0-5 sec: Free flow, no congestion

- 5-15 sec: Light delays (traffic lights)

- 15-30 sec: Moderate congestion

- > 30 sec: Heavy traffic or long signal