FCD.xml - Floating Car Data Output File

🚗 What is FCD.xml?

FCD stands for Floating Car Data - it's a detailed GPS-like tracking log of every vehicle's position and speed at every timestep during the simulation.

📊 What Does FCD.xml Contain?

Example FCD Output:

<?xml version="1.0" encoding="UTF-8"?>

<fcd-export>

<timestep time="0.00">

<vehicle id="test_bike_r1" x="-929.65" y="593.31" angle="90.00"

type="easyBike" speed="0.00" pos="0.00" lane="E0_0" slope="0.00"/>

</timestep>

<timestep time="0.50">

<vehicle id="test_bike_r1" x="-929.45" y="593.35" angle="90.12"

type="easyBike" speed="0.60" pos="0.30" lane="E0_0" slope="0.00"/>

</timestep>

<timestep time="1.00">

<vehicle id="test_bike_r1" x="-928.95" y="593.42" angle="90.15"

type="easyBike" speed="1.80" pos="0.90" lane="E0_0" slope="0.00"/>

<vehicle id="test_bike_r2" x="-932.80" y="590.33" angle="85.50"

type="easyBike" speed="0.00" pos="0.00" lane="E4_0" slope="0.00"/>

</timestep>

<!-- ... continues for every 0.5 seconds until simulation ends ... -->

</fcd-export>

Each Record Contains:

| Parameter | Unit | Meaning | Example |
| --- | --- | --- | --- |
| time | seconds | Simulation timestamp | 125.00 |
| vehicle id | - | Vehicle identifier | "test_bike_r1" |
| x | meters | X-coordinate position | -800.33 |
| y | meters | Y-coordinate position | 409.39 |
| angle | degrees | Vehicle heading (0-360°) | 90.00 (East) |
| type | - | Vehicle type | "easyBike" |
| speed | m/s | Current speed | 12.50 m/s |
| pos | meters | Position along current edge | 820.84 m |
| lane | - | Current lane ID | "E0_0" |
| slope | degrees | Road incline | 0.00 (flat) |

🎯 Why is FCD.xml Needed?

1. Vehicle Trajectory Analysis 🗺️

Track the complete path of every vehicle:

import pandas as pd

import matplotlib.pyplot as plt

# Load FCD data

fcd = pd.read_xml('fcd.xml')

# Plot trajectory of one vehicle

vehicle_data = fcd[fcd['id'] == 'test_bike_r1']

plt.plot(vehicle_data['x'], vehicle_data['y'])

plt.title('Vehicle Trajectory')

plt.xlabel('X Position (m)')

plt.ylabel('Y Position (m)')

plt.show()

2. Speed Profile Analysis 📈

See how speed changes over time:

# Speed over time for a vehicle

plt.plot(vehicle_data['time'], vehicle_data['speed'] * 3.6)  # Convert to km/h

plt.title('Speed Profile')

plt.xlabel('Time (s)')

plt.ylabel('Speed (km/h)')

plt.show()

3. Traffic Flow Visualization 🚦

- Where do vehicles slow down? (Congestion points)

- Where do they speed up? (Free flow zones)

- Identify bottlenecks in the network

4. Route Comparison 🛣️

Compare vehicle movements on different routes:

# Compare speeds on different routes

route1_vehicles = fcd[fcd['id'].str.contains('_r1')]

route2_vehicles = fcd[fcd['id'].str.contains('_r2')]

print(f"Route 1 avg speed: {route1_vehicles['speed'].mean() * 3.6:.2f} km/h")

print(f"Route 2 avg speed: {route2_vehicles['speed'].mean() * 3.6:.2f} km/h")

5. Animation and Visualization 🎬

Create animated videos of traffic:

- SUMO-GUI uses FCD data internally

- Export to visualization tools (QGIS, Google Earth)

- Create time-lapse animations

6. GPS-Based Studies 📡

Simulate real-world GPS tracking:

- Vehicle tracking systems

- Fleet management analysis

- Delivery route optimization

7. Accident/Incident Analysis 💥

- Identify collision points

- Study near-miss events

- Analyze traffic conflicts

8. Energy Consumption Correlation ⚡

Combine with battery.xml:

# Merge FCD with battery data

merged = fcd.merge(battery, on=['time', 'vehicle_id'])

# Energy vs speed analysis

plt.scatter(merged['speed'] * 3.6, merged['energyConsumed'])

plt.xlabel('Speed (km/h)')

plt.ylabel('Energy Consumed (Wh)')

plt.show()

🔍 FCD vs Other Output Files

| File | Focus | Data Frequency | Use Case |
| --- | --- | --- | --- |
| fcd.xml | Position & Speed | Every timestep (0.5s) | Trajectory, movement patterns |
| battery.xml | Energy & Battery | Every timestep (0.5s) | Energy analysis, battery SoC |
| tripinfo.xml | Trip Summary | Once per vehicle | Overall trip statistics |
| summary.xml | Network Stats | Every second | System-wide metrics |
| emissions.xml | Emissions & Energy | Every timestep (0.5s) | Environmental impact |

📏 File Size Comparison

For a 500-second simulation with 500 vehicles:

| File | Approximate Size | Records |
| --- | --- | --- |
| fcd.xml | 50-100 MB | ~500,000 records (1000 timesteps × 500 vehicles) |
| battery.xml | 30-60 MB | Same, but only for EVs |
| tripinfo.xml | 100-200 KB | 500 records (one per vehicle) |
| summary.xml | 50-100 KB | 500 records (one per second) |

FCD.xml is usually the LARGEST output file!

🎨 Practical Uses for Your Research

1. Route Efficiency Analysis

# Calculate average speed by route

route_speeds = fcd.groupby(['id']).agg({

'speed': 'mean',

'time': 'max'

})

# Which route is fastest?

route_speeds['avg_speed_kmh'] = route_speeds['speed'] * 3.6

print(route_speeds.sort_values('avg_speed_kmh', ascending=False))

2. Congestion Detection

# Find where vehicles slow down

slow_points = fcd[fcd['speed'] < 1.0]  # Speed < 1 m/s = stopped

congestion_locations = slow_points.groupby(['x', 'y']).size()

print("Top congestion points:")

print(congestion_locations.sort_values(ascending=False).head(5))

3. Travel Time Analysis

# Time spent in each road segment

for vehicle_id in fcd['id'].unique():

veh_data = fcd[fcd['id'] == vehicle_id]

edges = veh_data['lane'].str.split('_').str[0]  # Extract edge from lane

time_per_edge = veh_data.groupby(edges)['time'].agg(['min', 'max'])

time_per_edge['duration'] = time_per_edge['max'] - time_per_edge['min']

print(f"\n{vehicle_id} edge durations:")

print(time_per_edge)

4. Heatmap Creation

import seaborn as sns

# Create position heatmap

plt.figure(figsize=(12, 8))

plt.hist2d(fcd['x'], fcd['y'], bins=50, cmap='hot')

plt.colorbar(label='Vehicle Count')

plt.title('Vehicle Density Heatmap')

plt.xlabel('X Position (m)')

plt.ylabel('Y Position (m)')

plt.show()

5. Speed Limit Compliance

# Check if vehicles exceed speed limits

max_speed_limit = 13.89  # 50 km/h in m/s

speeding = fcd[fcd['speed'] > max_speed_limit]

print(f"Speeding violations: {len(speeding)}")

print(f"Percentage: {len(speeding)/len(fcd)*100:.2f}%")

⚙️ Controlling FCD Output

In Test1.sumocfg:

<output>

<!-- Basic FCD output (all data) -->

<fcd-output value="fcd.xml"/>

<!-- Filter by vehicle IDs -->

<fcd-output.filter-edges value="E0,E1,E2"/>  <!-- Only specific edges -->

<!-- Reduce file size: output every 2 seconds instead of 0.5 -->

<fcd-output.period value="2.0"/>

<!-- Geographic coordinates (if you have geo-referenced network) -->

<fcd-output.geo value="true"/>

<!-- Exclude stopped vehicles -->

<fcd-output.filter-shapes value="true"/>

</output>

Disable FCD (if you don't need it):

<output>

<!-- Comment out or remove this line -->

<!-- <fcd-output value="fcd.xml"/> -->

</output>

🚫 When You DON'T Need FCD.xml

You can skip FCD if you only need:

- ✅ Energy consumption data (use battery.xml)

- ✅ Trip summaries (use tripinfo.xml)

- ✅ Network-wide statistics (use summary.xml)

You NEED FCD if you want:

- ❌ Detailed trajectories

- ❌ Speed profiles over time

- ❌ Position-based analysis

- ❌ Traffic flow visualization

💡 Pro Tip: Convert FCD to Excel

Using the TraCI script I provided earlier:

# In the exporter, add FCD parsing:

def fcd_xml_to_excel(self, xml_file='fcd.xml'):

tree = ET.parse(xml_file)

root = tree.getroot()

data = []

for timestep in root.findall('timestep'):

time = float(timestep.get('time'))

for vehicle in timestep.findall('vehicle'):

record = {

'timestep_sec': time,

'vehicle_id': vehicle.get('id'),

'x_position_m': float(vehicle.get('x')),

'y_position_m': float(vehicle.get('y')),

'speed_ms': float(vehicle.get('speed')),

'speed_kmh': float(vehicle.get('speed')) * 3.6,

'angle_degrees': float(vehicle.get('angle')),

'lane': vehicle.get('lane'),

'pos_on_lane_m': float(vehicle.get('pos')),

'slope_degrees': float(vehicle.get('slope', 0))

}

data.append(record)

df = pd.DataFrame(data)

df.to_excel('fcd_output.xlsx', index=False)

return df

📊 Real Example from Your Simulation

For your 3 km route over ~4 minutes:

FCD will record:

- Every 0.5 seconds = 480 data points per vehicle

- 12 test vehicles = 5,760 position records

- ~500 flow vehicles = ~240,000 position records

- Total: ~250,000 GPS-like tracking points

This allows you to:

- Create complete trajectory maps

- Identify where EVs slow down (need charging?)

- Compare actual vs expected travel times

- Validate route choices

- Detect traffic bottlenecks

🎯 Summary

FCD.xml is:

- ✅ GPS tracking data for all vehicles

- ✅ Position + speed every 0.5 seconds

- ✅ Essential for trajectory analysis

- ✅ Useful for traffic flow studies

- ⚠️ Creates large files

- ⚠️ Optional if you only need energy data

For your EV battery research:

- Primary files: battery.xml, tripinfo.xml

- Supporting file: fcd.xml (for route validation)

- Optional: emissions.xml, summary.xml

You can disable FCD in Test1.sumocfg if you want smaller output files and only care about battery/energy analysis!